To get me to what I think was a flawless launch, without going with something more aggressive, like slicks...

- I will drop tire, pressure, gradually as I noted in my post from yesterday, I find that somewhere between 28 - 30psig, ends up yielding the best launch without impacting top end speed.

- I place car in dynamic mode "checkered flag"

- Shifter into Sport "S"

- Traction Control Fully Off, so hold down for 10 seconds or till it says DSC Off, see item #10 below.

- I will do a small burnout, to clean off any gravel/ dirt, and to get some heat in the tires

- I will stage the car (so both sets of light are on).

- Hold the brakes firmly with my left foot, and immediately start to bring up the revs to ~1000 - 1200rpm, I use sound to gauge where the engine is revving, I don't confirm with a visual check, as I'm too busy focusing on the lights.

- When the 2nd Yellow light illuminates, I release the brake, and slowly start rolling into throttle. It's a process best described, as you have to feel for traction, you simply keep giving it more and more, assuming you're not slipping. This is a case where less is more. Rev up too high, say 1500rpm or 1800rpm, and you will spin, roll into the throttle too quickly and you will spin (like I did with my 2nd run yesterday). If I can launch a RWD ~700HP and ~700lb-ft of torque (assumed figures at the crank given my WHP and Wheel Torque numbers), and register a flat 1.80 60 foot, where an AWD lighter car (F-Type) on the same tires registers a 1.78 60', that tells me I'm doing pretty good.

- Once your foot has gone gradually to the floor, stay in it, till you pass the end of the 1/4 mile, then let off, and start braking.

- If at any point, your back end starts to go squirrely/ let loose, let off immediately; do not brake till the car has straightened out.