Carefully wash the chassis. Remove all dirt and possible loosening of the underbody. Then the car must dry. Remove the side panel in the trunk.

For the available perforated holes, see the overleaf drawing. Drill holes (1) and (2) from the ground in the engine compartment, into the engine compartment support,(7) and (8) into the doors, (9) in rear door posts. From the front wheel arches (28), (29), (30) and [31) into the guards of the front mud and from the rear wheel arch respectively (18) and (19) into the frame carriers,

For treatment 'inside and outside approx. 5 liters Dinitrol

Spraying

Dinitrol ML, the special product for the inner rust protection, in hollow cavities such as carriers, tubing boxes and doors spray as well as on the chassis attachments, in joints and grooves and on the undersurface treated areas. Particularly careful and plentiful in the places which are designated red in the scheme. The result is not affected, when the surfaces are still somewhat moist. The temperature of the car must not be below + 10 ° C, as dinitrol will not penetrate properly.

Dinitrol Car is installed on the whole chassis and in the Wheelbox injected. Cover the disc brakes before spraying and Do not spray on motor and power transmission parts.

From flat ground

Dinitrol ML in the engine compartment by (1) and (2) spray, front and middle door posts through the. Shutter openings (3), (4} and (5),with reduced pressure. Carefully lower the front doors with reduced pressure through the openings of the door stoppers (6) and partly through (7). Rear doors through (8). Beam downwards towards the door. Rear door post (9).

From the trunk of sheet metal joints between the wheel housing and the outside plate (10} as well as the metal plates in the rear Fouling (11).

From lifting platform or lubrication pit

Remove all wheels. Spray with Dinitrol ML:

Front crossmember 		2-3

Chassis rails 		14-19, 20-25

Front cross member		26-27

Cavity in the front wings	28-29, 30-31

Align the beam so that the back of the metal sheet and the lining on the outer plate and the torpedo wall are treated. Also spray backwards and forwards against the door posts

Cross carrier above front jacking point 		32, 33

Sills	 						34-36, 37-39

Cross carrier above rear jacking point		 	40, 41

Rear cross-members 					42-43, 44-46

Box sections under trunk floor 			47, 48

Rear trunk box section 				49

(50) Joints and grooves as well as open sections spray, stiffening plates

at the front crossmember. Car body fasteners inside,

Flat pockets in the wings as well as above the gas tank.

Complete chassis with Dinitrol Car, also wheel box. Hub caps, moldings, etc. with Dinitrol ML

After-treatment as usual