Engine - Drive/Supercharger Belt Chirping Noise.

NUMBER JTB00045 (ISSUE 3) MODEL S-TYPE, XJ, XK, XK8 DATE 29 MAY 2009 ISSUE '3' CHANGES ARE HIGHLIGHTED IN GRAY SECTION: 303 Drive Belt Noise - Repair Procedure

AFFECTED VEHICLE RANGE

CONDITION SUMMARY:

Situation:

A noise or chirp may be heard from Front End Auxiliary Drive (FEAD) belt or Supercharger drive belt on cold start. This may be caused by the installed idler pulleys not controlling belt tracking correctly.

Action:

In the event of a customer concern of the above, refer to the Repair Procedure outlined below to:

| ^ | Normally Aspirated (NA) vehicles: replace the FEAD belt and idler pulley with new flanged idler pulley and belt. ^; Supercharged (SC) vehicles: replace the FEAD belt and idler pulley with new flanged idler pulley and belt; and / or replace supercharger idler pulley with new grooved idler pulley, tensioner, pulley bracket, and drive belt.. |
| --- | --- |