Upper Arm RH.

Upper Arm RH Removal - Remove the shock absorber and spring assembly. - Remove the cabin air filter. - Remove the cabin air filter housing retaining nut. - Remove the cabin air filter housing. - Remove the engine compartment support. - Remove the engine compartment panel. - Detach the wiring harness mounting bracket. - Remove the upper arm retaining nut. - Detach the engine compartment battery junction box. - Release the engine compartment battery junction box retaining clips. - Detach the engine compartment battery junction box. - Detach the hydraulic control unit. - Remove the upper arm retaining nut. - Remove the upper arm. Installation - Install the upper arm. - Install the upper arm retaining nut. - Install the upper arm retaining nut. - Install the shock absorber and spring assembly. - CAUTION: The final tightening of the Upper arm retaining nut must be carried out with the vehicle on its wheels. Tighten to 47 Nm . - CAUTION: The final tightening of the Upper arm retaining nut must be carried out with the vehicle on its wheels. Tighten to 47 Nm . - Attach the hydraulic control unit. - Attach the engine compartment battery junction box. - reposition the engine compartment battery junction box . - Attach the engine compartment battery junction box. - Attach the wiring harness mounting bracket. - Install the engine compartment panel. - Install the engine compartment support. - Install the cabin air filter housing. - Install the cabin air filter housing retaining nut. - Install the cabin air filter.