FUEL TESTS  -  2 x 2007 2.L TDV6 JAGUAR S-TYPE 6sp AUTO

Test Results:

CAR	FR RIMS	BK RIMS	TRIP Km	LITRES	Km/L	L/100Km	MPG (UK)	MPG (US)	VAR %

Test 1:

(13/01/2014)

KEN	18”x8”	18”x8”	522	33.9	15.40	6.49	43.49	36.22

DOUG	20”x9”	20”x10”	522	37.6	13.88	7.20	39.21	32.65	-  9.8 %

Test 2:

(03/02/2014)

KEN	20”x9”	20”x10”	521	37.1	14.04	7.12	39.66	33.03	- 6.9 %

DOUG	18”x8”	18”x8”	521	34.7	15.01	6.66	42.41	35.32

Notes:

both cars of same model, year, engine and similar service history – specifically checked prior to tests for friction impacting issues – ie lubrication oil, bearings, brake pads / rotors and EPB release, all windows up, no external drag items;

each car contained driver only with spare but no luggage or added load on board – one hour drive to start ensured DPF regen completed immediately before start;

both cars filled with identical fuel to full at approx 4am at PM Service Centre, then driven approx 260Km south on Pacific Hwy, then (after quick breakfast), returned to start point for refill and measure at same fuel bowser and volumes recorded;

tyre (tire) pressures were checked and adjusted to manufacturers' specs (+4lb for warm) on each car at PM Service Centre at start of each test;

cars were driven  in close proximity, but not within 50m of each other, and neither engaged in tailgating or slip streaming, either with each other or any other vehicle;

cruise control was engaged and used in each vehicle for identical periods and co-ordinated by the two drivers using phone communication between drivers;

as result of feedback on forum, the test was repeated to eliminate any car-specific variances in vehicles – the sole change between test 1 and test 2 was the exchange of wheels & tyres (tires) from one vehicle to the other for second test;

potential variance in driver acquired adaptive transmission control is acknowledged;

use of <http://www.convert-me.com/en/convert/fuel/> is acknowledged in converting metric fuel measures into UK and US averages.