Jaguar 32-bit ECM recovery procedure

2002-on X-TYPE, 2003-on S-TYPE, 2004-on XJ, 2003-on XK

In the event communications to the Engine Control Module (ECM) cannot be established or an error occurred

during programming and the ECM will no longer communicate with the WDS,

DO NOT replace the ECM.

Perform the ECM rectification procedure that will 'unlock' the ECM and will restore communications. Before continuing, have WDS in the docking station, the docking station plugged in and a battery charger on the car's battery.

1. Start programming the ECM as an existing module using the WDS.

During the reprogramming process, the screen in figure 1 will appear.

2. Disconnect the negative battery lead and press 'YES" on the

WDS screen as shown in figure 2.

3. Reconnect and tighten negative battery terminal and then press

'YES' to continue.

4. NOTE: Be aware of the time requirements of this portion of the

procedure. A helper may be required to perform correctly. Press

tick on WDS, wait 2 seconds, then switch ignition on. WDS will

recover and then reprogram ECM.