XJ

---

SALES Et MARKETING- :2013 MV XJ PRODUCT BRIEF

DATE: MODEL:

SUBJECT:

Ma'=J 23, 2012

XJ

2013 Model Year- Job#1Specifications

INTRODUCTION

The 2.012 Model Year focused primarily on the top of the XJ line, enhancing the rear environment luxury features of the XJ/XJL Supersport and introducing sport-oriented option packs on the Supercharged and Supersport derivatives.  For 2.013 Model Year, the focus is on powertrain actions designed to answer increased consumer demand for more efficient engines.

As announced at the Beijing Autoshow last month, Jaguar is launching two new engines, one of which - the new Supercharged 3.0-Jiter V6 -will be available on the XJ, raising to four the number of available powertrains on that model line- new V6 Supercharged,V8 Naturally aspirated and V8

Supercharged in both 470hp and 510hp versions. In addition, to further enhance the relevance of the product in the market place, all engines are fitted with an 8-speed automatic transmission and with Intelligent Stop/Start.

The XJ experience is further enhanced at 13MY by the introduction of two new, high quality audio systems, switching from Bowers & Wilkins TM to the Meridian™ brand.  And in response to retailers' request for more brand visual recognition, the JAGUAR 11 script has been added below the leaper on the center of the rear trunk lid.

As previously announced, at 2.013MY we are launching the stunning XJL Ultimate. For details regarding this new exclusive addition to the XJ line, please see the separate Product Brief that was issued on Apri12.2nd.

These changes and others affecting the XJ model range at 13MY launch are detailed in the Product

Brief that follows.

TIMING

•	Job #1 2.013MY- June 2.3, 2.012.

•	Dealer First Order call- anticipated mid-May 2.012.. Further information regarding retailer ordering will be issued separately by Distribution

•	Start of Sales - expected early Q4 2.012

TABLE OF CONTENT

1.	New Powertrain and Transmission

2..	Other feature changes for 2.013 MY

3·   Product Line up change summary

4·   Color, Trim & Veneer

5·   Wheels

Changes In standard  equipment, options or prices can occur. Jaguar reserves the right to change product specifications at any time without Incurring obligations.

1.   NEW POWERTRAIN AND OVMAMICS

1.1 New .Jaguar 3.0L V6 Engine

At launch, a new 3.0L V6 Supercharged engine is available on the standard wheel base XJ, raising to 4 the number of engines available on the car line (see section #4- Product Line-up change summary- for engines detailed application).  This engine is based on the architecture of the acclaimed Jaguar V8 engines, providing a refined and smooth power delivery and includes the following specifications:

•	Roots twin-vortex supercharger design: provides instant torque response and greater efficiency. Water cooled intercooler reduces intake air temperature optimizing power & efficiency.

•	Variable Camshaft Timing: Twin variable valve timing with intake and exhaust valve timing independently controlled, optimizing power, torque and economy at all engine speeds.

•		Direct Injection:  High-pressure (150bar) direct fuel injection system ensures precise fuel delivery for cleaner, more efficient combustion.  Enhanced droplet control and vaporization creates a homogeneous mixture greatly improving combustion.  Charge cooling effect results in higher volumetric efficiency and knock limit improvement.

•	Orientated Spark Plugs: spark plug orientation aligns the spark plug electrode with the direct fuel injection system, improving combustion robustness (misfire free) and minimizing spray impingement onto spark plug giving improvements in spark plug electrode cooling. Spark plug orientation control provides improved idle stability, dilution tolerance and spark plug durability.

•		New Balancer Gear: the engineering balance target of 75gcm within the V8 90deg bank angle required an innovative solution resulting in a pair of independently aligned counter rotating balance weights at both ends of the crankshaft

Changes in standard equipment, options or prices can occur.  Jaguar reserves the right to change product specifications at any time without incurring obligations.

Page 2 of 15

1.2 New B-speed Automatic: Transmission

All XJ engines are fitted with the ZF {8H P70) 8-speed automatic gearbox including lightweight rear differential with aluminum casing and improved bearings.   It offers the following improvements over the previous 6-speed unit:

•	Larger overall ratio and more even gear spacing providing an improved fuel economy at high speed and smoother acceleration.

•	Quicker shift timing- Gear shifts are initiated in just 200 milliseconds.

•	Multiple downshifts are enabled meaning direct downshifts are possible rather than sequential shifting.

•	Improved efficiency- Energy losses are kept to a minimum through the use of an improved pump design and improved control of gear elements at idle and in drive, helping to further improve fuel economy.

•	Weight control - Use of lightweight components has enabled the overall weight and dimensions of the gearbox to remain the same despite the extra gears and enhanced torque capability.

•	The transmission is operated in the same way as the previous 6-speed unit and continues to offer 'Sport' and 'Dynamic' mode allowing for manual gear shifts via the paddle shift. (see further information in the feature change section regarding paddle shift functionality in "D" and "S" mode)

Changes in standard equipment, options or prices can occur.  Jaguar reserves the right to change product specifications at any time without incurring obligations.

1.3 Intelligent Stop/Start

1.3.1. System Description

Intelligent Stop/Start is part of the Jaguar Land Rover sustainability strategy. The system detects when it is appropriate to shut down the engine- known as 'Eco Stop'- hence the 'intelligence' of the system. When the driver releases the brake pedal the engine re-starts almost instantaneously and the vehicle is ready to 'creep' or accelerate (Eco Start). The system is fully integrated with the

Vehicle architecture and specific vehicle systems can establish whether or not to allow an Eco Stop

and can request an Eco Start.

•	The Jaguar Intelligent Stop/Start system incorporates a TSS (Tandem Solenoid Starter) to achieve the Stop/Start functionality. A TSS system comprises an upgraded starter motor with tandem solenoids to achieve instant re-starts, regardless of the current status of the engine (engine stopped or on run down). This is known as 'change of mind' functionality which removes any hesitation in re-start, a problem which can afflict our competition.

•	An advantage of the TSS system and its 'change of mind' feature is the speed of engine re start - even if the engine is on rundown (i.e. if the engine has only just I has not yet stopped). The engine will re-start quicker than the time taken for the customer's foot to come off the brake pedal and press the accelerator pedal.

•	A secondary battery is fitted specifically for engine re-starting, meaning that all the normal car systems (radio, ventilation, lights, etc.) continue to receive power from the standard car battery and are unaffected by the re-start. In addition, a complex control system ensures that the Stop/Start operation only occurs when the car does not require the engine for other functions.

1.3.2 Intelligent Stop/Start Operation Criteria

There are a number of factors affecting whether or not Intelligent Stop/Start will function. Here are a few common factors whereby the system will not initiate an Eco Stop:

•	Ambient temperature: For driver comfort, Stop/Start only operates between 32°F and 104°F.

•	Engine temperature: If the engine is very cold, Stop/Start will not operate. The engine needs to be at the correct operational temperature for Stop/Start to function.

•	Cabin ambient temperature: The climate control system constantly monitors the cabin conditions and determines whether the engine is required to maintain customer comfort. For example, if 'Max Defrost' is selected, Stop/Start will not operate.

•	Vehicle is already moving or starts to move: The vehicle needs to be stationary for the Stop/Start system to operate. If the vehicle starts to move (>o.62mph) the engine will re start.

•	Seatbelt latching:  If the vehicle is stationary, the Stop/Start system will perform an 'Eco Stop' as soon as the driver unlatches the seatbelt and exits the vehicle, or as soon as the hood is opened. This is a unique feature of the system.

•	Vehicle power requirements: Conversely, the engine will perform an 'Eco Start’ if the vehicle detects it requires additional power to fulfill its function.

1.3.3 Driver Controls and Instrument Tell Tale for Intelligent Stop/Start

The default setting for Intelligent Stop/Start is 'on'.  With every ignition cycle, Intelligent Stop/Start will be turned on automatically. Intelligent Stop/Start can be turned off manually via a hard button located below the touch screen but it will be turned on again for the vehicle's next journey. Stop/Start operates in all modes apart from Reverse.

An illuminated 'ECO' symbol appears on the Instrument Panel when the engine has been stopped by the system. The engine re-starts as soon as the driver's foot releases all pressure from the brake

pedal.

From XF Instrument cluster

Turn on/off button

1.4 Peddle Shift Functionality's

At 2012MY, manual shifting can only be completed when sport "S" mode is engaged on vehicles fitted with naturally aspirated engines only. Recognizing that some drivers do want the ability to manually shift gear in Drive "D" mode while others are inconvenienced by the paddles position and the potential for  inadvertently hitting the shift paddles and selecting manual mode, at 13MY the paddle are now adjustable via the "Vehicle set up" in the instrument  cluster.

The vehicle default delivery setting is based on engine fitment

•	V6 Supercharged- paddles are not active in ''D" mode

•	V8 Naturally aspirated- paddles are not active in "D" mode

•	V8 Supercharged 470hp and 510hp- paddles are active in both “D" and "S" modes

To change the setting:

• Press the "Menu/OK" button on the vehicle steering wheel to display the list of options

• Select the "Vehicle Set-up" option and open the sub menu

• Select the "Gearbox Paddles" option and again open the sub-menu

• Setting of "Active in S only" or "Active in D & S" will be identified and can be changed as desired by the driver.

2-FEATURE CHANGES

2.1 Exterior Changes

A new exterior badging strategy is introduced on XJ:

•	New "JAGUAR" script on the center trunk lid below the leaper, helping to reinforce the brand recognition of the vehicle

•	"3.0" or "5.0" engine displacement badge on the right hand side of the trunk

•	"Supercharged”, Supersport" or "Ultimate" script is retained on the side power vents, however the "portfolio" script is discontinued

At 13MY, all XJ are fitted with a roof antenna pod similar to the XF.

2.2 Interior  Changes

•	At 13MY, the standard seats movements on all XJ derivatives except for XJ /XJL Supersport are being amended.   The XJ swb is fitted with 12x12 heated front seats while front seats massage becomes an option on XJL and XJ/XJL Supercharged. See table below summarizing the changes from 12MY to 13MY.

Seat Configuration Changes 2012 to 2013  Model  Year

|  | XJSWB |  | XJL Portfolio XJL Supercharged |  | XJ/XJL Supersport |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 12MV | 13MY | 12MY | 13MY | 12MY | 13MY |
| Standard Seat Fitment Fore/ Aft Cushion Height Squab recline Cushion tilt 4way lumbar Powered headrest Cushion extension Adjustable bolsters Massa.re | Driver Pass ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ | Driver Pass ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ | D&P ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ | D&P ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ | D&P ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ | D&P ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ ,/ |
| Total "Ways" | 16 12 | 12 12 | 20/20 | 18 | 20/20 | 20 |
| Other features Seat Material I P, door panels & upper fascia Headliner Heated Cooled Memory | Softgrain Soft grain Morzine ,/ ,/ ,/ ,/ ,/ | Softgrain Softgrain Morzine ,/ ,/ ,/ | Softgrain Softgrain Suede cloth ,/ ,/ ,/ | Softgrain Softgrain Suede cloth ,/ ,/ ,/ | Semi aniline Semi aniline Leather ,/ ,/ ,/ | Semi aniline Semi aniline Leather ,/ ,/ ,/ |
| Front Seat Massage Option |  |  |  | Opt |  |  |
| Front Massage |  |  |  | ./ |  |  |
| Total "Ways" with option |  |  |  | 20 |  |  |
| Front Seat comfort Pack |  | Pck Pck |  |  |  |  |
| Powered headrest Cushion extension Adjustable bolsters (2W) Front Seat Massage Cooled Front Seats |  | ,/ ,/ ,/ ,/ ./ ./ ,/ ,/ ,/ ,/ |  |  |  |  |
| Total "Ways" with Pack |  | 20 20 |  |  |  |  |
| Portfolio Pack (swb) 0 Pck Pck |  |  |  |  |  |  |

•	New rear seat pack

The rear seat comfort pack that was launched as standard equipment on the XJL Supersport at

12MY is now available on all LWB derivatives, in combination with the executive pack on the XJL

and as a separate package on XJL Supercharged.

| XJL Rear Seat Comfort Pack | XJL Supercharged Rear Seat Comfort Pack |
| --- | --- |
| • Rear seat power recline (outboard seats recline independently) • Rear seat 4-way lumbar • Rear seat massage • Rear seat footrest • Winged headrest on both front and rear seats • Front passenger seat away* • Rear business Tray • Electric rear window sunblind • Premium front and rear floor mats • Intaglio with Union Jack and "Jaguar" script • Chrome side mirror covers | • Rear seat power recline (outboard seats recline independently) • Rear seat 4-way lumbar • Rear seat massage • Rear seat footrest • Winged headrest on both front and rear seats • Front passenger seat away* |

* The front passenger seat can be adjusted forward, via control accessible by both the driver and the rear right hand passenger, to allow for increase rear seat legroom for the rear passenger-side occupant.

•	A new compact key originally scheduled to launch at 12MY is planned for 13MY introduction

2.3	Infotainment- Meridian Audio System

Meridian™ Audio is an ultra-premium British audio specialist widely regarded as the audiophiles' choice offering a multi award-winning range of audio equipment globally. For 13MY, new MeridianrM audio systems will be standard on all XJ

2.3.1	Meridian™ Audio Unique Design and Technology

• Total system optimization

-   Speakers and amplifiers are perfectly matched to optimize efficiency and sound quality.This achieves better audio quality with lower power requirement than conventional systems.

• Highest sound quality

- 		Other manufacturers compare the sound of their systems to others in their class. Meridian™ use real instruments such as a Steinway Piano or the human voice as their benchmark. The result is that Meridian™ speakers are truer to the original sound.

• Psychoacoustics

- 		Meridian engineers have utilized research into psychoacoustics to produce sounds that are so true to life that they are perceived by the brain as the real thing giving listener greater connection with the sound inside the car.

2.3.2 Meridian™ Audio features

• Digital Signal Process (DSP) software- All elements of the system are optimized

• Dynamic Volume Control - Uses a front-mounted microphone to detect variations in cabin noise and compensates for changes in ambient sound levels to provide consistent sound quality.

• Meridian Cabin Correction

- 	Employs advanced DSP to complement the acoustics of the vehicle, improving clarity and authenticity of sound

- 	Meridian Audio's room correction algorithms have been incorporated to overcome problem

of acoustic resonances within the cabin.

- 	Digital filtering technology is used to ‘smooth’ these resonances ensuring that the crucial pitch and timing of transient details is accurately reproduced.

• Meridian Digital Dither Shaping- Digital signal conversions and processes are executed accurately and smoothly throughout the system, maintaining maximum detail ensuring highest quality from all sources.

• Meridian Trifield System- Delivers a perfect stereo sound to all occupants in the vehicle. It

enhances the performance of stereo material by using three front channels to increase localization accuracy and rendering a more stable, natural soundstage.

2.3.3 Audio System Power Output

Traditionally, the performance of automotive audio systems has been measured in terms of power output in Watts. Meridian adopts a different view, designing the speakers to work harmoniously with the amplifier as an integrated, active audio system. This approach provides a superior system without requiring the high power required by many audio systems.

Total Harmonic Distortion (THD) of an audio signal is a measure of the harmonic distortion. Harmonic Distortion occurs when a signal is "clipped" or altered due to a part of the sound system not being capable of reproducing the signal. If the system runs out of headroom the signal distorts, producing undesirable sound due to limitations in the amplifier design. A lower level of THD will allow the components in a loudspeaker & amplifier to produce a more accurate audio reproduction by reducing the distortion added by electronics and audio media. A THD rating less than 1% is

considered to be high-fidelity and inaudible to the human ear. The audio systems used before 13MY have a THD of 10% where the new Meridian audio systems have a THD of only 0.2%. This reduction in harmonic distortion means a lower power output is required to achieve the high levels of audio clarity within the vehicle.

2.3-4	Two levels of Meridian audio system are available on the XJ

| Meridian™ Premium Audio System | Meridian Premium 7.1 Surround Audio System |
| --- | --- |
| • 380W premium Sound System • 12 Amplifier Channels • 12 Speakers + 2 subwoofers • Separate DSP • Audyssey MultEQ sound equalization • Meridian Cabin noise correction • Dynamic volume control Standard on: XJ and XJL Portfolio | • 825W premium Sound System • 15 amplifier Channels • 18 Speakers+ 2 subwoofers • Separate DSP • Audyssey MultEQ sound equalization • Meridian Cabin noise correction • Dynamic volume control • Meridian Trifield System Standard on: XJ/XJL Supercharged XJ/XJL Supersport XJL Ultimate Optional on: XJ & XJL Portfolio |

3. PROOUCT ONE UPCHANGE SUMMARV

TWO  WHEELBASES,  FOUR TRIM LEVELS, AND FOUR ENGINES

The XJ Continues to be offered in both standard and long wheel base variants. Four trim levels are available:

•	Base trim- offers soft grain leather seats with heated front and rear seats.

•		"Portfolio" trim -adds contrast stitching and piping, embossed headrests, soft grain leather door panels, instrument panel and fascia,suedecloth headlining,18/18 way adjustable front seats, cooled front and rear seats and 4-zone climate control

•	"Supersport" trim - adds semi aniline leather seats, soft grain leather headliner, lighted air

vents and door/trunk sills and offers inlaid wood veneers and tipped leather options

•		"Ultimate" trim- Adds full partition leather trimmed rear center console, powered passenger table, integrated illuminated beverage chiller, seat back iPad® docks with sliding door and charging capability and unique trim color, veneer and exterior color.  For further details on this exclusive trim, please see the Product Brief issued on April 22, 2012

Engine applications:

•	At launch the standard wheel base variants are available with the new Supercharged V6, the Supercharged V8 470hp or the Supercharged V8 510hp. The V8 Naturally aspirated engine is no longer available on standard wheel base XJ.

•	Identical to 12MY, the long wheel base variants are available with the V8 Naturally aspirated

385hp, the Supercharged V8 SC 470hp or the Supercharged V8 510hp.

Standard 	Engine 	Trim Level 	Long 	Engine 	Trim Level

WheelBase	Wheel Base

XJ 	V6SC 	Base trim 	XJL Portfolio	V8NA 	Portfolio

340hp 	385hp

XJ Supercharged 	V8SC	Portfolio	cjo 	XJL Supercharged 	V8SC	Portfolio	c/o

470h 	470h

XJ Supersport 	V8SC	Supersport 	c/o	XJL Supersport 	V8SC	Supersport 	c/o

510h	510h

XJL Ultimate	V8SC	Ultimate 	nja

510h

Veneer Fitrnent- All models except XJ/XJL Supersport and XJL Ultimate

Veneer

Burl Walnut Satin Elm Figured Ebony Rich Oak

Piano Black

Carbon Fiber

Image	Fitment and Availability

XJ (standard) XJL Portfolio

XJ/XJL Supercharged

XJ

XJL Portfolio

XJ/XJL Supercharged

XJ w/ Port Pk (standard) XJL Portfolio (standard) XJ/XJL Supercharged

XJ/XJL Supercharged

All

Optional on all

($1,500)

Available with Wood Steering Wheel

Yes

Yes

Yes

Veneer Fitrnent -X.J/X.JL Supersport

Supersport Veneers

Image

Fitment and Availability

Available with

Wood Steering

Wheel

Walnut Curl Piano Black Satin Zebrano

Shadow Walnut

New for13MY

Supersport 	Yes

All

Supersport

Supersport 	Yes

Rich Oak with

Linear Laser Inlays

Figured Ebony with

Ribbon Laser Inlays

Carbon Fiber

Supersport

(standard)

Supersport

Supersport ·

(no cost option)

Yes

Yes

X.J Ultirnote - Unique Herringbone Veneer Standard- See separate Product Brief

S.WHEELS

•	The 20" Amirante wheel style is no longer available at 13 Model Year

•		Naturally aspirated models (XJ and XJL) come standard with 19" wheels with all-season tires; a selection of 20" wheels on summer/performance tires are available as options. These models, regardless of wheel/tire, have a top speed limited to 121mph

•	Supercharged and Supersport models come standard with 20" wheels with

summer/performance tires; these models have a top speed limited to 155mph; 19" wheels

with all-season tires are an option on Supercharged and Supersport models- when equipped

with 19" wheels these models have a top speed limited to 121mph

Finish:

Size  I Style I Finish

19" Aleutian

Fitment and Availability

Standard on: XJ

- Sparkle Silver

Optional on: all

Finishes:

19"Toba

Standard on: XJL Portfolio

(Sparkle Silver)

- Sparkle Silver

-Polished

Optional on: all

Finishes:

20" Kasuga

Standard on:

XJ & XJL Supercharged

(High gloss Silver)

- High Gloss Silver

-Polished

20" Mataiva

Finish:

-Anthracite with chrome detailing

20" Orona

Finishes:

- Anthracite with diamond turned spokes

-Polished

Optional on: all

Standard on:

XJ & XJL Supersport

Optional on: None- Supersport only Standard on:

None - option only

Optional on: all

Finish:

2o"Venom

Standard on:

Sport & Speed/Speed Pack

-Painted with Diamond turned spokes

Optional on:

None - Sport & Speed/Speed pack only

6. 	RESOURCES

Order Guide

For additional reference- including technical specs, the full list of standard and optional equipment, color & trim chart and wheels- see the separate 2013 MY XJ Order Guide, available on the Product & Distribution website: https:f/jaguarvehicles.jlrext.com/newxjlindex.html

Online Car Configurator

To visually see and build the various color and trim combinations of XJ available visit the online Car Configurator on the JaguarUSA.com website, which will be updated in August with 2013MY: http:L/www. jaguar. com/us/en/#/buildyourjaguar

If you have any additional product questions please contact your Market Manager or me. Marine Prache

Product Manager, Jaguar Sedans

E: mprache@jaguar.com

T: (201) 818-8281

C: (201) 815-1295