JAGUAR XJ – TECHNICAL DATA

|  | XJ 3.0L V6D 600 |  |  | XJ 5.0L V8 N/A |  | XJ 5.0L V8 S/C |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ENGINE & TRANSMISSION |  |  |  |  |  |  |  |
| Engine Capacity (litres) | 3 |  |  | 5 |  | 5 |  |
| Engine Capacity (cc) | 2993 |  |  | 5000 |  | 5000 |  |
| Cylinders | 6 |  |  | 8 |  | 8 |  |
| Valves per Cylinder | 4 |  |  | 4 |  | 4 |  |
| Compression Ratio | 16.0:1 |  |  | 11.5:1 |  | 9.5:1 |  |
| Bore Stroke/mm | 84.0/90.0 |  |  | 92.5/93.0 |  | 92.5/93.0 |  |
| Bore Stroke/inch | 3.30/3.54 |  |  | 3.64/3.66 |  | 3.64/3.66 |  |
| Transmission Final Drive Ratio | 2.73:1 |  |  | 3.31:1 |  | 3.31:1 |  |
| Transmission | 6-speed automatic |  |  | 6-speed automatic |  | 6-speed automatic |  |
| 1st | 4.17 : 1 |  |  | 4.17 : 1 |  | 4.17 : 1 |  |
| 2nd | 2.34 : 1 |  |  | 2.34 : 1 |  | 2.34 : 1 |  |
| 3rd | 1.52 : 1 |  |  | 1.52 : 1 |  | 1.52 : 1 |  |
| 4th | 1.14 : 1 |  |  | 1.14 : 1 |  | 1.14 : 1 |  |
| 5th | 0.87 : 1 |  |  | 0.87 : 1 |  | 0.87 : 1 |  |
| 6th | 0.69 : 1 |  |  | 0.69 : 1 |  | 0.69 : 1 |  |
| Rev | 3.40 : 1 |  |  | 3.40 : 1 |  | 3.40 : 1 |  |
| PERFORMANCE |  |  |  |  |  |  |  |
| Fastest 0-60 mph (seconds) | 6.0 |  |  | 5.4 |  | 4.7 |  |
| Fastest 0-100 kph (seconds) | 6.4 |  |  | 5.7 |  | 4.9 |  |
| Top Speed (mph/kph) | 155/250 |  |  | 155/250 |  | 155/250 |  |
| Max. Power (SAE bhp) @ rpm | n/a |  |  | 385@6500 |  | 510@6000-6500 |  |
| Max. Power (EEC PS) @ rpm | 275@4000 |  |  | 385@6500 |  | 510@6000-6500 |  |
| Max. Power (EEC kW) @ rpm | 202@4000 |  |  | 283@6500 |  | 375@6000-6500 |  |
| Torque (SAE lb-ft) @ rpm | n/a |  |  | 380@3500 |  | 461@2500-5500 |  |
| Torque (EEC Nm) @ rpm | 600@2000 |  |  | 515@3500 |  | 625@2500-5500 |  |
| PS per Litre of Engine Displacement | 92 |  |  | 77 |  | 102 |  |
| Aerodynamics (Cd) | 0.29 |  |  | 0.29 |  | 0.29 |  |
| ECONOMY | SWB |  | LWB |  |  |  |  |
| Combined Fuel Economy (mpg) | 40.1 |  | 39.2 | 24.9 |  | 23.4 |  |
| Combined Fuel Economy (l/100km) | 7.0 |  | 7.2 | 11.3 |  | 12.1 |  |
| US C-H Combined (mpg - US) | n/a |  | n/a | 19 (LWB) / 18 (SWB) |  | 17 |  |
| NEDC Combined CO2 (g/km) | 184 |  | 189 | 264 |  | 289 |  |
| Fuel Tank Capacity (litres) | 82 |  | 82 | 82 |  | 82 |  |
| Fuel Tank Capacity (USA gallons) | n/a |  | n/a | 21.7 |  | 21.7 |  |
| Emissions | EUV |  | EUV | ULEVII/TIERII BIN5/EUV |  | ULEVII/TIERII BIN5/EUV |  |
| WEIGHT | SWB | LWB |  | SWB | LWB | SWB | LWB |
| EU: Kerb Weight (lbs) | 3960 | 3998 |  | 3870 | 3909 | 4172 | 4223 |
| EU: Kerb Weight (kg) | 1796 | 1813 |  | 1755 | 1773 | 1892 | 1915 |
| Federal: Lightest Weight (lbs) - actual kerb weight | n/a | n/a |  | 4045 | 4131 | 4281 | 4323 |
| Federal: Lightest Weight (kg) - actual kerb weight | n/a | n/a |  | 1835 | 1874 | 1942 | 1961 |
| Gross Vehicle Weight (lb) | 5215 | 5215 |  | 5072 | 5149 | 5215 | 5270 |
| Gross Vehicle Weight (kg) | 2365 | 2365 |  | 2300 | 2335 | 2365 | 2390 |
| DIMENSIONS | SWB | LWB |  | SWB | LWB | SWB | LWB |
| Boot (litres) | 520 | 520 |  | 520 | 520 | 520 | 520 |
| Boot (cu ft) | 18.4 | 18.4 |  | 18.4 | 18.4 | 18.4 | 18.4 |
| Wheelbase (inches) | 119.4 | 124.3 |  | 119.4 | 124.3 | 119.4 | 124.3 |
| Wheelbase (mm) | 3032 | 3157 |  | 3032 | 3157 | 3032 | 3157 |
| Front Track (inches) | 64 | 64 |  | 64 | 64 | 64 | 64 |
| Front Track (mm) | 1626 | 1626 |  | 1626 | 1626 | 1626 | 1626 |
| Front Overhang (inches) | 35 | 35 |  | 35 | 35 | 35 | 35 |
| Front Overhang (mm) | 890 | 890 |  | 890 | 890 | 890 | 890 |
| Rear Track (inches) | 63.1 | 63.1 |  | 63.1 | 63.1 | 63.1 | 63.1 |
| Rear Track (mm) | 1604 | 1604 |  | 1604 | 1604 | 1604 | 1604 |
| Rear Overhang (inches) | 47.2 | 47.2 |  | 47.2 | 47.2 | 47.2 | 47.2 |
| Rear Overhang (mm) | 1200 | 1200 |  | 1200 | 1200 | 1200 | 1200 |
| Overall Length (inches) | 201.7 | 206.6 |  | 201.7 | 206.6 | 201.7 | 206.6 |
| Overall Length (mm) | 5122 | 5247 |  | 5122 | 5247 | 5122 | 5247 |
| Overall Width (inches) mirrors folded | 74.6 | 74.6 |  | 74.6 | 74.6 | 74.6 | 74.6 |
| Overall Width (mm) mirrors folded | 1894 | 1894 |  | 1894 | 1894 | 1894 | 1894 |
| Overall Width (inches) incl. mirrors | 83.1 | 83.1 |  | 83.1 | 83.1 | 83.1 | 83.1 |
| Overall Width (mm) incl. mirrors | 2110 | 2110 |  | 2110 | 2110 | 2110 | 2110 |
| Overall Height (inches) | 57 | 57 |  | 57 | 57 | 57 | 57 |
| Overall Height (mm) | 1448 | 1448 |  | 1448 | 1448 | 1448 | 1448 |
| Driver/Passenger Leg Room (inches) | 42 | 42 |  | 42 | 42 | 42 | 42 |
| Driver/Passenger Leg Room (mm) | 1055 | 1055 |  | 1055 | 1055 | 1055 | 1055 |
| Rear Leg Room (inches) | 39 | 44 |  | 39 | 44 | 39 | 44 |
| Rear Leg Room (mm) | 987 | 1121 |  | 987 | 1121 | 987 | 1121 |
| DRIVING DYNAMICS | SWB | LWB |  | SWB | LWB | SWB | LWB |
| Tightest Turning Radius Kerb to Kerb (feet) | 40.4 | 41.7 |  | 40.4 | 41.7 | 40.4 | 41.7 |
| Tightest Turning Radius Kerb to Kerb (m) | 12.3 | 12.7 |  | 12.3 | 12.7 | 12.3 | 12.7 |

Document updated February 2010.