# Jaguar XJ8 Battery Disconnect Instructions

- Insure that no key is in the ignition.

- Insure the security system is disarmed by unlocking the doors.

- Remove luggage compartment floor panel.

- Disconnect negative (-) lead (Ground Terminal) first.

- Slacken the negative lead pinch bolt and disconnect the negative (-) battery lead.

- Slacken the positive lead pinch bolt and disconnect the positive (+) battery lead.

# Jaguar XJ8 Battery Reconnection Instructions

- Insure that no key is in the ignition.

- Do not over tighten the pinch bolts.

- Slacken the negative lead pinch bolt and disconnect the negative (-) battery lead.

- Clean the battery posts and coat the post with petroleum jelly.

- Ensure the vent tube is connected and free from kink or damage.

- Connect the positive (+) lead, tighten the pinch bolt and clip the positive terminal cover on the battery.

- Connect the negative (-) lead and tighten the pinch bolt.

# After Reconnecting the Battery

- After the battery has been reconnected and the ignition switch is turned to position II, the message center display, will initially show an erroneous reading i.e. - - - - - - . This will remain until the bulb check sequence has completed, when the correct recorded mileage will be displayed. This is a normal function of the instrument cluster.

- Reset the electric parkbrake.

  - The message APPLY PARKBRAKE will be displayed when the ignition is switched on.

  - Depress the footbrake and pull the parkbrake switch up to apply the parkbrake. This is required to reset the parkbrake system which will now function properly.

- Reset the electrically operated windows anti-trap function.

  - Fully close the window, hold the switch in the close position for 2 seconds and then release the switch. Wait for 2 seconds, do not open the window.

  - Again, select the same window to close, holding the switch for a further 2 seconds.

  - Check to confirm the operation by opening the window and then use the one-touch operation to close the window.

  - Repeat steps a through c for the remaining electrically operated windows.

- The radio will not operate until the correct security code has been re-entered.

  - Code will appear on the display. Enter the code (----). When entering the code, if a “beep sound” occurs begin the code entry over, starting with the first digit. This “beep” was provided to indicate the starting point for the code entry.

  - If the code is not properly entered in 3 successive attempts the radio will not accept any entry until the radio has been left on for 1 hour. After 1 hour in the “on” position the radio will accept 3 additional entries.

- Reset the analog clock on the dashboard to the correct time.