Cooling System Backflushing

CAUTION: The heater core must be backflushed separately from the engine

cooling system to prevent the engine cooling system particles from clogging the heater

core tubes and reducing (or eliminating) coolant flow through the heater core. All engine

cooling system flushing and backflushing procedures must include a separate

backflushing of the heater core after the flushing or backflushing of the engine cooling

system.

CAUTION: Heater core internal pressure must not exceed 100 kPa (14.5 psi).

Failure to follow this instruction may cause damage to the heater core.

NOTE:

Cooling system backflushing should be carried out before the cooling system components

are installed after the cooling system flushing procedure.

1. Disconnect the heater outlet coolant hose from the engine and connect the heater hose to a

suitable hose pipe.

2. Disconnect the heater inlet coolant hose from the engine and allow the coolant to drain into

a suitable container.

3. Turn the water supply valve to the hose ON and allow water pressure to flow through the

heater core.

4. Allow water pressure to flow through the heater core for approximately five minutes.

5. Turn the water supply valve to the hose OFF and disconnect the hose pipe from the heater

hose.

6. Connect the heater inlet coolant hose to the engine.

7. Connect the heater outlet coolant hose to the engine.

8. Fill the cooling system as described using a 50% mixture of Jaguar Premium Cooling

System Fluid, or equivalent meeting Jaguar specification WSS M97B44-D and 50% distilled

water.

9. Test the system for correct heater performance with the specified engine cooling system

conditions.

Radiator Backflushing

1. Remove the radiator.

2. Invert the radiator.

3. Connect a suitable hose pipe to the lower coolant hose connection of the radiator.

4. Turn the water supply valve to the hose ON and allow water pressure to flow through the

radiator.

5. Allow water pressure to flow through the radiator for approximately five minutes.

6. Turn the water supply valve to the hose OFF and disconnect the hose pipe from the

radiator.

7. Allow the coolant to drain from the radiator.

8. Install the radiator.

Engine Backflushing

1. Remove the thermostat before backflushing the engine.

2. Position the high-pressure water hose into the engine through the engine return and

backflush the engine.

3. Connect a suitable hose pipe to the upper coolant hose connection of the engine.

4. Turn the water supply valve to the hose ON and allow water pressure to flow through the

engine.

5. Allow water pressure to flow through the engine for approximately five minutes.

6. Turn the water supply valve to the hose OFF and disconnect the hose pipe from the upper

coolant hose connection of the engine.

7. Connect the upper coolant hose to the engine.

8. Fill the cooling system as described using a 50% mixture of Jaguar Premium Cooling

System Fluid, or equivalent meeting Jaguar specification WSS M97B44-D and 50% distilled

water.

9. Test the system for

1. Remove the radiator.

2. Invert the radiator.

3. Connect a suitable hose pipe to the lower coolant hose connection of the radiator.

4. Turn the water supply valve to the hose ON and allow water pressure to flow through the

radiator.

5. Allow water pressure to flow through the radiator for approximately five minutes.

6. Turn the water supply valve to the hose OFF and disconnect the hose pipe from the

radiator.

7. Allow the coolant to drain from the radiator.

8. Install the radiator.

Engine Backflushing

1. Remove the thermostat before backflushing the engine.

2. Position the high-pressure water hose into the engine through the engine return and

backflush the engine.

3. Connect a suitable hose pipe to the upper coolant hose connection of the engine.

4. Turn the water supply valve to the hose ON and allow water pressure to flow through the

engine.

5. Allow water pressure to flow through the engine for approximately five minutes.

6. Turn the water supply valve to the hose OFF and disconnect the hose pipe from the upper

coolant hose connection of the engine.

7. Connect the upper coolant hose to the engine.

8. Fill the cooling system as described using a 50% mixture of Jaguar Premium Cooling

System Fluid or equivalent, meeting Jaguar specification WSS M97B44-D and 50% distilled

water.

Vehicles with 3.5L or 4.2L engine

Item Part Number Description

1 — EGR valve

2 — Throttle body

3 — Heater core

4 — Water pump

5 — Engine bank (left-hand)

6 — Thermostat

7 — Engine oil cooler

8 — Lower hose

9 — Coolant expansion tank

10 — Radiator

11 — Auxiliary coolant flow pump

12 — Upper hose