Jaguar Xj6 x300: Hi my jaguar x300 has a starting problem but

Resolved Question:

Hi my jaguar x300 has a starting problem but only when it's hot outside, it takes ages to start but when it's cold outside it starts straight away , any ideas ?

Submitted: 3 years ago.

Category: UK Car

Share this conversation

Mike

Expert:  Mike replied 3 years ago.

Hello and Welcome to justanswer..

How long have you had this issue for ...? Has it just started to occur...? Has any work been done on the car...anything that might have a bearing on this?

Has anything been checked / done etc - any parts checked / replaced and so on so far in a bid to cure this?

The more you can tell me, the better...but to inc the answers to the above at least -

Once I receive your reply, I will get back to you asap thereafter..

Kind Rgds, MIKE........................................

Ask Your Own UK Car Question

Customer: replied 3 years ago.

Hi mike I've had the car for 6months and I've had the problem when ever the weather is hot, the only work I've done is to change the water pump and alternater , I gave the car a full service but the starting problem remains. Regards geoff

Expert:  Mike replied 3 years ago.

Hello again Geoff.. Thank you for your reply & info..

Assuming then that all is well service wise, there are two suspects here, known things that will cause the problems you have / describe..

Both are sensor issues.. sometimes when these , either go faulty, they will put a fault light/ message on the dash - but not always, so you cant rely on that. If you do have a fault message coming up on the dash, then that means the Pcm (main computer ) has detected the fault and would have logged it meaning a plug in diagnostic should be done to reveal the cause... If no such light/ message, its not worth spending on having a diagnostic done as there will be nothing there..

Assuming you have no fault light then, the first of the two sensors in question is the Ect sensor...the engine coolant temperature sensor, which tells the ecu the temperature of the engine - the ecu / pcm then adjusts the sensors/ actuators /injection etc for the engine to start properly -per its temperature at any given time. If and when this sensor goes faulty, it will cause staring problems such as you describe. So, this is the most likely fault here. The Ect sensor is easily replaced, and not expensive. I advise get a genuine Jaguar supplied one as aftermarket ones can be troublesome. I have uploaded a picture for you to help you ID the sensor - for that, click HERE..

Given your symptoms, Im sure you will find a new Ect sensor will cure this issue for you.. The other sensor as I referred to above, for IF the Ect sensor wasn't to cure it or fully cure it, is the crank position sensor. A pic for that and its location( front of engine ) I have uploaded for you HERE. That again would then be , on the face of things, the next most likely sensor that would give you the problems you have .. but Im not an advocate of just changing parts .. The Ect sensor is such a common fault that causes this problem..and again , its not expensive so its always worth fitting a new one straight off.. but.. IF you dont have an improvement, then I strongly recommend having a plug in diagnostic test done to see whats going on. Not so much for fault codes but Live Data testing , is the way then to find the fault and cure it..

So, go with the Ect sensor first and all being well, that will see it completely sorted...there is a 99% chance of that..but if you do still have troubles - follow through as above. Most any place has the diagnostic equipment to run tests on these - you dont have to use a dealer..so if needed, that is a cheaper option for you ...

I sincerely XXXXX XXXXX above all helps you and helps with the understanding of the problem you have -and what needs to be done to cure this ...

Please feel free to get back to me if you need / wish to discuss this more ..( I will always get back to you as soon as I receive any reply you may send .. To reply to me, use the "reply to expert" tab or link... . Meantime, please do rate the answer I have given for you with the OK service or higher... for which I thank you in advance... as thats how Im credited for helping you ..

Again - feel free to get back to me at any time ...even if you need more help as time goes on - clicking the rating as above does not close you out.. Thank you again...

Kind regards, MIKE.

Mike

Mike, UK Car Mechanic

Category: UK Car

Satisfied Customers: 39185

Experience: Hi, I am a Car/ vehicle technician of 34 years. 29of -running my own repair/diagnostic business.

Mike and other UK Car Specialists are ready to help you

Ask your own question now

Share this conversation

JustAnswer in the News:

What Customers are Saying:

This expert is wonderful. They truly know what they are talking about, and they actually care about you. They really helped put my nerves at ease. Thank you so much!!!!

Alex

Los Angeles, CA

< Previous | Next >

Read more: http://www.justanswer.com/uk-car/6yor7-jaguar-xj6-x300-hi-jaguar-x300-starting-problem.html#ixzz3nkxWgVOP I sold the Jaguar in 2012 so this site is not maintained

Engine Control Unit

I am a private owner; use this information at your own risk

The crank position sensor is always one of the first things to suspect should the engine not start - I carry a spare in the boot as they are so often the cause of failure. Several have suggested that they should be a routine replacement at 100,000 miles or ten years whichever comes first. (further information below)

Engine Control Unit problems

The car had been on the drive, just below freezing for several days, with a light covering of snow. Car had had no problems starting or running prior to this.

Started fine and went to the local shops.  On way home the car started normally but started misfiring and wouldn't exceed 2,000rpm and eventually backfired.  Got home OK but obviously not right!   Tickover was normal.

With inconsistent symptoms it initially felt like a water-in-the-fuel problem; or maybe fuel regulator or maybe fuel pump or filter or pipework.

To eliminate these I bought and fitted a Fuel Pressure Test Kit from this seller - currently eBay item 250771928164  Simple and effective; I bought the back entry 60psi gauge and have left it in place permanently.

If the engine won't run then by bridging across the pump relay contacts (blue relay in the boot fusebox) showed a steady fuel pump current of 5.4amps and fuel pressure at the injector rail of 44psi with engine off.

The fuel pressure gauge in the flexible pipe should show a steady 38psi with the engine running, this varies a little with inlet vacuum as the engine is revved up.

Just for completeness - a rogue MAF (Mass Air Flow meter) can give idle dip and lumpiness and/or a hot restart issue.  Also check for air leaks in the inlet bellows and be aware that a faulty throttle assembly (throttle not closing fully due to gumming up and/or faulty ISCV (Idle Speed Control Valve) can cause a high idle speed.

To reset the ISCV (Idle Speed Control Valve):

- with the valve off the throttle body and the ignition on you will eject the whole spindle out of the body. The sequence is get someone to switch on the ignition to position 1 then tell them to switch off once the valve has retracted, failure to do this and the valve will then wind forward looking for the valve face, as it is not there it keeps on winding out.

- do not just wind in the spindle into the body you will damage the location of the spindle in the body.  With spindle/plunger, sleeve and spring assembled wind into the body slowly when the location just starts to enter the body get a eye glass and align the flat/groove on the spindle with the matching section in the body, then get said person to switch on the ignition, then switch off when the spindle has fully retracted.

High idle has been mentioned several times but a forum search will tell you that you DO NOT need the system reset at a dealer, that is only needed when a NEW TPS is fitted. The system is a closed loop system and gets it's information from the ecu and crankshaft position sensor to maintain idle speed.

First try a replacement valve to correct the idle fault, if that cures it - job done.

If not do the basics: first strip and clean throttle body, clean manifold face and throttle body face's to ensure flat and clean, BIG point - there is no gasket and it is NOT a metal to metal seal, apply .5-1mm diameter bead of black Hylomar automotive sealant to throttle body face around ISCV channel and throttle bore insert 2 top bolts, align onto manifold then push body onto manifold hand tighten bolts insert 2 bottom bolts tighten all bolts assemble all parts, then retest, now although the Jaguar CD does not mention the above use of sealant for the assembly it is an approved sealant by Jaguar.

Also suspected a dying Crankshaft Position Sensor - a fairly common fault.

check that the Tachometer needle moves when cranking, if it does then move on, if not then could be the Crankshaft sensor. Stuck out on front of the engine, this tends to fail with age but can give a no-spark condition at any time. These sensors do fail completely but can also partially fail and give confusing and inconsistent problems from no-start to intermittent firing and restricted rev range.  In my case the engine started and ticked over no problem most times but would not rev beyond 2,000 rpm.

The original sensor tested OK - with resistance of 1,250 ohms and inductance of 1 henry. However the replacement sensor seemed to cure the problem - almost!  The engine would now rev to 3,000rpm - more than enough power to drive around with.

But it wasn't right and still misfired.  Things slowly got worse - to undriveable - and eventually unstartable.

Having read on the Jag Lovers Forum that the wiring to the sensors could occasionally go faulty, I decided to remove the ECU which is behind the drivers side footwell side cover.  It is necessary to pull a little of the door seal away, starting at the
tread plate, and prise out the fir tree fixing in the centre of the panel.  Then slide the cover towards the back of the car to disengage the two clips which may fall off.

Two bolts hold the ECU in place but then (for me) things got difficult as the two 36-way ECU plugs were covered by a security strap help on by clutch head anti-tamper screws.  The head of this type of screw has a normal drive face and a chamfered rear face which prevents the screwdriver gaining any purchase when trying to undo the screws.

With limited space to manoeuvre, a few minutes work with my trusty angle grinder and the stupid strap was now off and I could get down to fault tracing.

I really wonder how Jaguar did their fault tracing if they couldn't take the plugs & sockets apart?  Presumably they gave the customer a new car if the ECU went faulty ?

Just as well I persisted - looking at the ECU on the bench I discovered that the black connector has attracted a small amount of water (melted snow ?) and caused serious corrosion.

Over a week or so this drop of water had formed conductive copper sulphate (I think, it was blue anyway) which had partially shorted some of the ECU pins together.

The connector insert was then removed and showed the conductive mess on both sides.

With the insert removed, the true damage could be seen. What a mess; no wonder the car eventually wouldn't go at all!

When cleaning, corroded pin 18 fell off so something had to be done

It is worrying to discover that Jaguar thoughtfully placed both ECU connectors on top of the ECU to ensure that any water would run down into the connector.

Eventually this is almost certain to happen, especially if you have a sunroof and the drains get blocked the overflow runs down the OUTSIDE of the pipes conveniently onto the wiring above the ECU

NOW we know the reason for the strap - it is to prevent anyone taking the plugs out and discovering this design fault.

the red connector was fine - having no sign of water ingress  :-))

These are the pin-out connections for the two ECU connectors.  I bought two 36 way Econoseal ECU PCB connectors from Pole Vault -  http://tinyurl.com/63mahvn

I checked the Throttle Position Sensor and Crankshaft Position Sensor wiring for correct resistances looking back into the wiring from the ECU connectors - both were OK  :-))

Crank Position Sensor resistance = 1.282K (note: this is not conclusive as the CPS output DOES deteriorate with time)

Throttle Position Sensor:  pin 7-11 = 3.98K fixed;   pin 7-12 = 528R-3.27K smooth;   pin 11-12 = 3.60K-600R smooth (further comment about the TPS at the bottom of this page)

Temperature sensor: part no: LHE1600AA
Iced water = 5,360 ohms    Room temperature = 2,800 ohms    Boiling pan of Water = 195 ohms

So - replaced the 36-way black socket on the ECU (don't try this unless you know what you are doing inside electronic modules).

All this complexity just to replace the carburettor and distributor  . . . . . .  ? and the butterfly and cam position sensor are still there!  Is this really progress ?

the ECU was not too bad inside - some dry powder from somewhere (aluminium oxide maybe) but after cleaning should be OK

Refitted the ECU and it worked :-))   Thought all problems were solved; replaced the footwell cover and - no start; completely dead.  Probably a dry joint (36 connections both sides of the PC board; maybe one didn't take).  :-((

The ECU's are SAGEM-LUCAS GEMS6 for a 3.2L N/A auto

The original failed ECU was a (1995) LNA1410BF/211 - 2795.    I have now fitted a (1996) LNA1410YD/211 - 2896 which went straight in and worked perfectly   :-)

The BF version is shown as superseded and the next one along is the YD, so should be OK.   The 211 stands for the automatic version (212 = manual).    I've had no problems since and I didn't recalibrate anything.

The 1935 Austin Ten hadn't been started for several months and had been covered in snow and ice, the same as the Jaguar.  Fired up right away of course.  Well you can't beat progress  . . . . :-))

There is some uncertainty whether the ECU needs to be reset by the Jaguar software when you change the TPS

I did it the other way around - fitted a 1996 ECU into my 1995 car when the original got wet. Didn't alter anything and it seems to work perfectly and no measurable change in fuel consumption.  Later - consumption seems to have improved from 18mpg overall to 20mpg overall - short journeys on narrow roads in hilly rural area  :-)

An ECU costing £1,600 should surely be able to sort out something simple like a change in the TPS surely ? After all, if it can recognise and adjust the timing when the camshaft sensor is set way out it should be able to recognise max and min TPS values?  And surely the TPS itself will be subject to some form of quality control ?

I stand to be corrected by an expert - maybe it is understandable that Jaguar would play safe by recommending paying the dealer just to check that everything was within tolerance - but is it essential ?   Anyone know for certain?

Have just come across the following from Speedypal:

The EMS  (Engine Management System) includes software that is what they refer to as adaptive.  We are interested in the TPS adaptive idle voltage.

This adaptive software learns the value of the voltage coming from the TPS when the idle speed is correct.  Depending on where the TPS was installed, the voltage will be a little different from where it was with the old TPS.  And that's OK!!  The adaptive software is designed to compensate for that and other changing conditions as well.

Providing that you got the TPS in approximately the right position the EMS will, within a short period of time (i.e., a few days) learn what the output voltage of the new TPS is when the idle speed is correct.  Pretty cool really .  But like much of the software in the world, there are situations that are not anticipated (imagine that) and in very rare instances it screws up.

The voltage supplied to the TPS potentiometer is 0 volts and +5 volts The wiper arm of the pot never reaches either of those rails and ends up some place around 5% to 10 % from the mechanical extremes.

That's assuming the TPS was installed in close to the correct position,  If it is close enough, the adaptive software will learn the new positions and all will be well again.

and from Steve (Jags-on-Gas)

The TPS is driven (rotated) by a tang much like the end of a standard screwdriver. Because of that it can be wrongly installed, 180 degrees out. I have seen one that read over 4.5V at idle, the result of the fitter rotating it before putting the bolts in.

To ensure correct installation you should see around 0.5V or around 10% on the scanner at idle. You can check the voltage on the green wire with yellow bands using a multimeter. The engine does not have to be running to do this check (better that it isn't). Just have the ignition on, look for 0.5V at idle position and around 4.5V full throttle.

# Starting problems on x300 4.0

|  | LinkBack | Thread Tools | Search this Thread |
| --- | --- | --- | --- |

#1 05-13-2013, 12:31 PM

Paul 1947 Junior Member; Join Date: May 2013 Location: swansea Posts: 5 Thanks: 0 Thanked 0 Times in 0 Posts.

Starting problems on x300 4.0 Hi can anyone help me, i have a x300 4.0 1996, it takes a lot of cranking over to start on a cold/hot engine, but when it fires into life it runs perfect with no heistation or missing, just a bad starter. i have replaced plugs, front timing pully sensor, throttle postion sensor, idle sensor air/fuel filter. No change, is there anyone who can advise problem.

#2 05-14-2013, 01:23 PM

GGG; Join Date: Dec 2011 Location: Durham, UK Posts: 75,163 Thanks: 1,712 Thanked 6,117 Times in 5,270 Posts.

Welcome to the forum Paul, I've moved your question from UK Region section to X300 section. Members here with the same model will be able to advise on starting problems. When you get a minute, please follow this link New Member Area - Intro a MUST - Jaguar Forums - Jaguar Enthusiasts Forum to the New Member Area - Intro a MUST section and post some info about yourself and your vehicle for all members to see. In return you'll get a proper welcome and some advice on posting to the forum. Graham __________________ 2007 XK 4.2 Coupe

#3 05-14-2013, 02:14 PM

sparkenzap Veteran Member; Join Date: Nov 2007 Location: atlanta ga Posts: 3,629 Thanks: 332 Thanked 781 Times in 647 Posts.

Paul: Believe it or don't... but the X-300 engine will try both crank orientations relative to the cams if the cam position sensor is faulty. It trys one, then the other rotation for spark. You might very well have a failed or improperly oriented CPS. Thats the one that looks like a short distributer and mounts on the front right of the engine where the distributer used to go. __________________ Ross MY 89 V12 convertible MY 99 XJR MY 02 XJ8 MY 12 XF R

| The Following User Says Thank You to sparkenzap For This Useful Post: |  |
| --- | --- |
| RJ237 (05-14-2013) |  |
| #4 05-15-2013, 01:39 AM |  |
| Paul 1947 Junior Member; Join Date: May 2013 Location: swansea Posts: 5 Thanks: 0 Thanked 0 Times in 0 Posts. |
| re x 300 starting problems Ross, many thanks for your reply, i will try changing that component, and see what happens, if no change i think its best to get the car on a diagnostic machine, once again many thanks Paul |  |

#5 05-15-2013, 03:14 AM

sparkenzap Veteran Member; Join Date: Nov 2007 Location: atlanta ga Posts: 3,629 Thanks: 332 Thanked 781 Times in 647 Posts.

Paul: Of course, the other well known issue for these engines is failing ckps - the crank position sensor. Although I have not seen it reported, a possible failure mode would be dodgy starting due to bad pickup at low crank speeds, but apparently they usually fail as the engine heats up. See if the tach registers at about 250 rpm while cranking- if so, it probably IS NOT the ckps. Good luck. __________________ Ross MY 89 V12 convertible MY 99 XJR MY 02 XJ8 MY 12 XF R Last edited by sparkenzap; 05-17-2013 at 06:22 AM.

#6 05-17-2013, 03:57 AM

Paul 1947 Junior Member; Join Date: May 2013 Location: swansea Posts: 5 Thanks: 0 Thanked 0 Times in 0 Posts.

hi ross, i have tried reversing the sensor in case it had been put on 180 degres incorrect, no difference, in fact it made no difference at all even when the sensor was not connected, still started after a lot of crancking, and runs beautifully, so the problem lies with that component one way or another, perhaps it couild be the ecu unit itself, back to e bay searching for bits i am afraid. thanks paul

#7 05-17-2013, 04:33 AM

SleekJag12 Senior Member; Join Date: Dec 2012 Location: Valley of the Sun, USA Posts: 538 Thanks: 62 Thanked 133 Times in 116 Posts.

Agreed, Ross... Paul, In my experience, the slightest rotational adjustment of the cam position sensor has a major effect on starting ability. But once running, all seems normal. Was the sensor disturbed (moved) before the problem began? Have you tried rotating it very slightly one way or the other, then trying again? Not exactly a scientific method, but probably worth a shot. __________________ Richard The XJ duo: 1996 Carnival Red XJ6 4.0L 1995 Flamenco Red XJ12 6.0L Gone but not forgotten: 1996 Spruce Green VDP6 4.0L

#8 05-17-2013, 06:38 AM

sparkenzap Veteran Member; Join Date: Nov 2007 Location: atlanta ga Posts: 3,629 Thanks: 332 Thanked 781 Times in 647 Posts.

Paul: In addition to Richard's advice, I would be suprised that the car has not thrown a code. Is the "check engine light" on? The device is a Hall effect sensor, so you should probably NOT use a test light, but rather a meter or scope. The black with brown stripe wire is the pulsed signal, and the black / green wire is the ground , according to the MY 95 manual. Attached Thumbnails __________________ Ross MY 89 V12 convertible MY 99 XJR MY 02 XJ8 MY 12 XF R Last edited by sparkenzap; 05-17-2013 at 06:48 AM.

#9 05-19-2013, 02:50 AM

Paul 1947 Junior Member; Join Date: May 2013 Location: swansea Posts: 5 Thanks: 0 Thanked 0 Times in 0 Posts.

re starting probs on x300 4 litre hi , yes the engine light does come on, so it must have thrown a code, how can that be rectified? ps, thanks for all your valuable i nformation and help paul

#10 05-19-2013, 02:54 PM

sparkenzap Veteran Member; Join Date: Nov 2007 Location: atlanta ga Posts: 3,629 Thanks: 332 Thanked 781 Times in 647 Posts.

You need to get the code read. Here in the US, many auto parts stores will read the codes for free. It takes less than 15 minutes for someone to do it, so any mechanic with a code reader should be able to do it for not too much $$. Get the code number (like P-220). The generic descriptions that come up on the scanner may not be right for a Jaguar, but the number tells the tale. __________________ Ross MY 89 V12 convertible MY 99 XJR MY 02 XJ8 MY 12 XF R

#11 05-19-2013, 03:09 PM

Paul 1947 Junior Member; Join Date: May 2013 Location: swansea Posts: 5 Thanks: 0 Thanked 0 Times in 0 Posts.

thanks Ross, i really appreciate your help, without it i would have been running around like a headless chicken, regards Paul

#12 05-20-2013, 03:21 AM

Ziggy Senior Member; Join Date: Jul 2012 Location: UK Posts: 331 Thanks: 25 Thanked 39 Times in 36 Posts.

Paul, you can get a code reader suitable for engine codes, for notalotacash. Mine's a bluetooth one that talks to my phone, and cost about £15! P.S: it would indeed take about 15 minutes if you included 14.5 minutes for a cup of tea... __________________ 1995 X300 3.2 & a few BMWs - but don't tell anyone!

| Similar Threads |  |  |  |  |
| --- | --- | --- | --- | --- |
| Thread | Thread Starter | Forum | Replies | Last Post |
| Gas tanks overheating problem | alpecsa | XJ6 & XJ12 Series I, II & III | 17 | 09-07-2015 11:55 AM |
| Today's electrical glitches! | tberg | XK / XKR ( X150 ) | 15 | 09-07-2015 07:32 AM |
| My introduction | David Thompson | New Member Area - Intro a MUST | 7 | 09-03-2015 07:48 PM |
| Stumbling under WOT | toronadomike | XK8 / XKR ( X100 ) | 5 | 09-03-2015 02:27 PM |
| !! RESTRICTED PERFORMANCE !! - Please HELP! | XFR_Gold | XF and XFR ( X250 ) | 2 | 09-02-2015 12:24 PM |

Tags

cam, car, cars, crank, crankshaft, current, ignition, jaguar, pauls, position, problems, sensor, start, x300, xjr

« Previous Thread | Next Thread »

Currently Active Use