Electronic Engine Controls - Engine Control Module (ECM) Adaptation

General Procedures

When to carry out adaptations

DTCs P1000/P1111

Conditions for adaptation

Fuelling Adaptations Drive Cycle

• NOTE: Make sure the engine is at normal operating temperature. (75°C, 167°F)

1. Whenever the battery is disconnected, the Engine Control

Module (ECM) adaptations are reset to a "green" ECM

condition. In this condition, certain functions of the ECM are

inhibited, and the ECM must "relearn" these functions. To

enable the ECM to do this, the vehicle will have to perform

some maneuvres which it may, or may not, do during a normal

road test. The routine in this procedure should be carried out at

Pre-Delivery Inspection, and after any emissions related

rectification work involving the disconnection of the battery.

2. When the DTCs relating to a rectified concern are cleared,

P1000 (system checks not complete since last memory clear)

will be flagged. This DTC does not indicate a failure, just that

diagnostic drive cycles need to be performed. The vehicle's onboard

diagnostic system will confirm that all the diagnostic

drive cycles are complete by flagging P1111, (system checks

complete since last memory clear)

1. Make sure these conditions are checked and corrected, if

necessary, before carrying out the adaptation procedure.

Mechanical Electrical

 Engine oil level correct

 Coolant level correct

 Fuel level greater than

30%, less than 80%

 Fuel filler cap correctly

fitted/secure, seal in good

condition

 All rectification work

relating to DTCs completed

 Fuel, ignition and cooling

systems in good order

 All Diagnostic Trouble

Codes (DTCs) cleared

 All rectification work

relating to DTCs

completed

 Fuses

 Harnesses

 Electrical connectors

1. Either the Jaguar approved diagnostic system (where

available), or a scan tool should be connected to measure the

Mass Air Flow rate. Most generic scan tools should be capable

of reading this parameter.

Depending onElectronic Engine Controls - Engine Control Module (ECM) Adaptation

General Procedures

When to carry out adaptations

DTCs P1000/P1111

Conditions for adaptation

Fuelling Adaptations Drive Cycle

• NOTE: Make sure the engine is at normal operating temperature. (75°C, 167°F)

1. Whenever the battery is disconnected, the Engine Control

Module (ECM) adaptations are reset to a "green" ECM

condition. In this condition, certain functions of the ECM are

inhibited, and the ECM must "relearn" these functions. To

enable the ECM to do this, the vehicle will have to perform

some maneuvres which it may, or may not, do during a normal

road test. The routine in this procedure should be carried out at

Pre-Delivery Inspection, and after any emissions related

rectification work involving the disconnection of the battery.

2. When the DTCs relating to a rectified concern are cleared,

P1000 (system checks not complete since last memory clear)

will be flagged. This DTC does not indicate a failure, just that

diagnostic drive cycles need to be performed. The vehicle's onboard

diagnostic system will confirm that all the diagnostic

drive cycles are complete by flagging P1111, (system checks

complete since last memory clear)

1. Make sure these conditions are checked and corrected, if

necessary, before carrying out the adaptation procedure.

Mechanical Electrical

 Engine oil level correct

 Coolant level correct

 Fuel level greater than

30%, less than 80%

 Fuel filler cap correctly

fitted/secure, seal in good

condition

 All rectification work

relating to DTCs completed

 Fuel, ignition and cooling

systems in good order

 All Diagnostic Trouble

Codes (DTCs) cleared

 All rectification work

relating to DTCs

completed

 Fuses

 Harnesses

 Electrical connector

1. Either the Jaguar approved diagnostic system (where

available), or a scan tool should be connected to measure the

Mass Air Flow rate. Most generic scan tools should be capable

of reading this parameter.

Depending on road conditions and dealer location, it may be

difficult to achieve the road speeds necessary to reach the air

flow indicated. It is possible to reach these air flow figures by

inducing a load on the vehicle, (making use of hills, using a

lower gear ratio, etc) or, if no alternative is possible, by

restraining the vehicle and reaching the air flow figure

Site

Mass Air Flow

(grams/second) Driving Conditions

1 Up to 7 g/s Idle in P for 2 minutes

2 10 to 14 g/s Open throttle in P for 2 minutes

3 18 to 22 g/s Open throttle in P or D with

guidance * for 2 minutes

4 25 to 30 g/s Drive with guidance * for 2

Diagnostic Monitor Completion

stationery in D.

WARNING: This should not be attempted without the vehicle

being in a safe area and suitably chocked. Failure to follow this

instruction may result in personal injury.

Without using the Jaguar approved diagnostic system or a scan

tool with system readiness capability, it will not be possible to

confirm when the adaptations are complete, unless P1111,

(system checks complete since last memory clear) is flagged. If

such equipment is not available, ALL the diagnostic monitors

will have to run to enable DTC P1111 to be flagged. In some US

states, this code is a requirement for annual emissions tests.

1. The drive cycles detailed here will enable the monitors to run,

but only after fuelling adaptations are complete.

Entry Conditions

Diagnostic

Engine

Speed

Vehicle

Speed

Mass Air Flow

(grams/second)

Time to

Completion

Catalyst Monitor  1100 to1600 rpm

50 to 72 kph (31 to 45 mph)

5 to 30 g/s 23 seconds

HO2S 600 to 2500 rpm

29 to 113 kph (18 to 70 mph)

4 to 16 g/s Fuel cut for

more than 10 seconds, but less than 30 seconds *

Catalyst Monitor Sensor

N/A N/A 10 to 90 g/s 140 seconds

Fuel System N/A N/A More than 6 g/s 33 seconds

AACV N/A 61 to 121 kph (38 to 75 mph)

4 to 16 g/s Fuel cut for 2 seconds *

P/N switch 1800 to 2200 rpm

80 to 101 kph (50 to 63 mph) N/A 31 seconds

Evaporative emissions

N/A 48 to 89 kph (30 to 55mph)

and idle

Refer to drive

cycles for leak

tests in this section

2. During the drive cycle, drive gently, avoiding hard acceleration,

braking or cornering and rough roads.

1. Drive at 84 to 97 kph (52 to 60 mph) for 40 seconds.

2. Accelerate to 113 kph (70 mph).

3. Close the throttle, (ideally at the start of a descent)

continue with closed throttle for 10 seconds. (* Fuel

cut)

4. Drive at 56 to 64 kph (35 to 40 mph), holding the

throttle as steady as possible, on a flat level road for