# Bluetooth

https://www.jaguarforums.com/forum/xj-xj8-xjr-x308-27/factory-phone-11547/

I managed to communicate with Jim Roal and he confirmed that the phone (PSE, Transceiver) module is not needed. Here is a summary of my findings:

Solution 1:

* Use the Jaguar Phone Module and Bluetooth Module if you want functioning steering wheel and radio buttons when making/taking calls.

* Pair to "Jaguar" using ##3#*# and PIN code 1313.

Solution 2:

* Use a Motorola/Parrot/Nokia/etc Bluetooth kit.

* You need a 10-pin 2-row connector to connect the Bluetooth kit to the Radio Telephone connector RT2 under the armrest.

* You won't be able to use the steering wheel and radio buttons when making/taking calls.

* Pair to your Bluetooth device using PIN code 0000.

Solution 3:

* Jabra SP700 or Jabra Cruiser SP710 Bluetooth In-Car Speakerphone

* Alpine EX-10

* There should be many other alternatives out there.

I wanted a quick, simple, and inexpensive fix thus chose the Motorola T605 and it works just fine. Motorola has the new TK30 that looks good. Parrot might be preferred by many; they have several great models with and without LCD. Nokia has the CK-15W, CK-600 and other models.

Based on much reading on these forums, here is some info that might help.

Phone/PSE Module: Need it to store the address book, so can do without it.

* LJC 7301AA | C2C25674 (superceeded by C2C34515) | 2W93-19K350-AC or AD or AE. I read it is advised not to consider version AA on this module.

* Here is one on ebay Germany (Telefon - Modul Jaguar XJ8 X308 LJC7301AA Autotelefon | eBay) that will cost you some $120.

Bluetooth module:

* C2C28506 (superceeded by C2C39062, C2C31974) | 2W93-14B409-AE – X350

* I read you can get at the dealer for some $200.

http://www.jagspares.co.uk/Manners/partdetail.asp?partno=C2C28506

The dealer will tell you if you need other components. I read some might need a D2B optical cable to hook the PSE into the car's fiber-optic system.

2W93 is the platfortm number for the XJ. Yours might have a different platform number. Check on that please.