LUCAS DISTRIBUTOR DATA (Distributor RPM   Distributor degrees)

P/N /   Model 	   rot'n      RPM 1 Adv 1 ± 1°   RPM 2 Adv 2 ± 1°    RPM 3 Adv 3 ± 1°  no advance below vacuum   years, make, models.

40617A/D   DMBZ6A   CW            2000 11     	   850 8      	 450 1.25     	  325     	  6/20/4.5         1961-71  Jaguar   XKE 3.8 & 4.2

41060A     22D6            CCW         2300 9.5     	   800 7     	 525 1.5      	  300   	   7/14/8            1966-69  Jaguar/Daimler   4.2 MK X and 												E Type, Sovereign 4.2

Vacuum advance units

The First number represents the vacuum point (in inches of mercury) at which the advance unit begins to operate.

The Second number gives the vacuum point at which the unit is at its full advance.

The Third  number gives the total advance (in Distributor degrees) for the unit.