Jaguar/Lucas 2 wire amplifier wiring system diagram.

Jaguar wiring is typically one wire gauge smaller than shown in this GM layout. (12>14, 14>16, 16>18 Gauge.)

Jaguar XJ6 S3:  1.    C to (-) terminal of Ignition Coil =        White with black stiped wire

2.    B to (+) terminal of Ignition Coil =       White wire

3.    IGN SW to (+) terminal of Coil =         Brown (or White) wire

4.    EFI to (-) terminal of Ignition Coil =     White with Black wire

5.    Tach to (-) terminal of Ignition Coil =  Slate with Blue spiral stripe wire

Condenser can be mounted on Ignition coil bracket mount bolt, with the + terminal of condenser hooked directly to positive terminal of Ignition coil. Wire from B to condenser is eliminated.