Hello again Clarke;

Yahoo ... the weekend is here ... and I made it!!! Several "late nighters" had me feeling my age, but I have enjoyed catching up on all the news.

I'm glad to see you have linked up with Grant and Graham. Cameron (Cambo351) is another great resource down under, as is Gus with his mass of brilliant download data at <jagrepair.com>.

You have been hard at work in your garage I see, and not just with the Triumph and Jeep. Given your multi carb experience with bikes, the Jag's HS8s will be quite straightforward. You are absolutely correct about then being 1 jet. Better still, there are only really 2 elements in play, although your gentleman John would be correct to point out there are several subsets of issues.

First, a quick background:

Each SU has a butterfly spindle whose sole function is to regulate air volume passed to the engine inlet - it has no direct impact on fuel metering, jets etc - and to regulate that air at idle, it is fitted with a finely adjustable idle adjustment screw.

The round alloy cylinder that is a feature of SU contains a piston with passages to allow induction vacuum to transfer into the carby cylinder, thus drawing up the piston under the restraint of its own weight + internal spring + internal oil damper. The lower piston face contains a long tapered needle, which regulates fuel delivery from an attached fuel bowl via a hollow stem into which the needle protrudes. The greater the air volume and velocity passing through the carb, the higher the piston, the thinner the cross-section of needle remaining in the single hollow jet - therefore, the greater the fuel delivery. Generally, manual chokes consisted of cable device which produced richer starting mixture by mechanically lowering the single hollow jet of each carb relative to the carb needle.

SU produce a vast number of needles with different, carefully manufactured tapers. - but the important element to grasp is that the chosen needle should provide variations in fuel mixtures that ideally suit all driving conditions likely to be encountered in each car. Jaguar spent considerable effort to perfect their recommended pairings to balance performance with good engine wear and reasonable economy.

Step 1 - Check & Dismantle:

Best advice I received is to ensure, before you start anywhere near the carbs, that all other engine issues are fully sorted. This should be seen as the final stage in engine tuning. OK?

Note - all parts removed must be kept separate so that they can be re-installed into their correct carb. Multiple plastic ice-cream containers are useful. Next, remove air duct from air cleaner filter to carbs, so that you can reach into the throat of each SU. Remove central oil damper. Gently lift lower edge of piston to full height, then allow to drop. Piston should drop quickly and smoothly with a pronounced "thonk" as it hits bottom. Any hesitation would indicate that the needle and jet tube are binding, wear will be inevitable, and both should be replaced.

Remove air pot retaining bolts. Carefully lift and remove the air pot cylinders, and remove piston spring. Even more carefully, lift and remove the piston - being most careful not to bump protruding needle. Loosen needle retention bolt and remove needle. Clean all parts in fuel and dry with lint free cloth.

Step 2 - Starting from Zero:

Loosen but do not remove throttle linkages, but do not alter the idle stop screws at this stage. Now, turn to the fuel chambers, ensuring that the float, needle and seat (cheap to replace if worn) maintains the fuel levels at the same height from carb to carb, Now, adjust choke cables and linkages to ensure the each jet tube returns to a flush position, as measured inside the bridge area on the floor of the carb body. My advice is not to dismantle the jet tube unless you have leakage, wear (as per initial check) or other issues here.

Next, zero the mixture adjustment nut surrounding jet under carb. Then wind each back equally the prescribed number of turns. Re-seat the needle; check that the needle shoulder is flush with piston face, and fix retention bolt. Re-check, adjust, fix. Carefully lower piston into carb. Remember, the jet may not now be concentric with needle. Next, stand springs side-by-side for visual check of free height. Assemble springs to their respective pistons. Fit air pot cylinders and retaining bolts. Carefully check lift and fall of piston for pronounced "thonk" or adjust jet concentricity if required.

Step 3 - Tuning:

And that's where I stop (unless you insist, LOL) - I figure you will have it well covered in the resources you have already picked up. Sorry this ended up so long winded, but I have tried to write it as I would now walk myself through it.

I have heaps to get done, so I had best go get at it Clarke.

Best wishes.

Ken