HE Tune up, Grant’s way, for those that want it.

There are some pre-tune checks that MUST be 100% prior to doing this, and there are NO shortcuts.

1) Make sure the throttle rods etc are all set CORRECTLY, coz if they are out at idle the damn thing will never be right.

2) Make sure the TPS signal is between 0.32v and 0.36v, if it is not the ECU will NOT drop into “closed loop idle mode”.

3) You have already un-seized the mechanical advance inside the distributor, RIGHT. If not, you are wasting your time, so do so before driving this beast anymore.

4) The spark plugs are gapped at 0.025”. If you don’t know, TAKE THEM OUT and gap them, or replace them with correctly gapped plugs, and do not forget the anti-seize on the threads.

5) The AAV is closing 100%, MMMMMM.

6) The vac advance capsule is working correctly, they do “toast” real well in the “V”.

7) The throttle discs are CLEAN of all that “black goo” that builds up periodically, if in doubt, clean them and check the clearance is set to 0.003” each side.

Set the “static” timing at 10 deg BTDC, this will be close enough, BUT make sure you got plenty of adjustment in the advance direction.

Mixture adjustment, yeh right. There is an adjustment in the side of the case of the ECU, and it only adjusts the mixture at IDLE (hot), after that it’s on its own with the sensors etc. There is a separate write up on “Adjusting the ECU fuel pot”.

Leave the vac hoses etc all attached, it is how you are going to drive it anyway.

With the engine warmed up, move the distributor adjusting screw until the engine starts to sound “sweet”, and STOP. Don’t go too far, you don’t have to move it much, coz there is only 30deg between cylinders. Now go for a drive, take the screwdriver and ½” spanner with you, stop at the curb, check the mirror, take off HARD, BUT BE PREPARED TO BACK OFF REAL QUICK, if it “pings. If it pings, retard the timing a “bit”, if it don’t “ping” advance it a BIT, then do it again, until you can take off hard without “pinging”. I usually have the a/c ON, as it “loads” the engine, and effects timing settings significantly.

I have NEVER timed one the Jag way, what a waste of time. You go to all the trouble of setting it at 3000rpm, and lay under the front, with it jacked up, with all that shit spinning real quick right in front of your nose, I am NOT that brave, and then drive it and it “pings”, so what do you do, back it off until it don’t ping, duh.

When you get the timing right, slowly turn that mixture screw on the ECU, and again, you will “feel” the engine “sweeten up” when it’s right. Now you may want/need to adjust this ECU a bit from time to time, as fuel qualities at the pump vary with seasons.

Idle speed is subjective, but around 650rpm in P or N is OK, some like it higher, it purely is choice. The idle is adjusted by that 13mm bolt head on the side of the AAV, I reckon you knew that, just covering it in case. Don’t mess with this adjustment until the engine is HOT. It is a waste of time until the AAV is closed 100%.

Timing adjustments are not required after you get it set, there are NO wearing components inside the distributor, I have not touched mine since 2003, when the engine rebuild was finished. I just remove the dist cap once a year, to lube the centre shaft, that’s all.

The only thing you will need to keep an eye on is the “gunk” that builds up around the throttle discs. The first sign of this is roughish idle, clean them with solvent (carby cleaner or similar) on a clean rag. I usually do mine every oil change, which is 4 monthly.