De-smogging/sorting a V12 HE.

This is an ILLEGAL procedure in some markets, so PLEASE check YOUR area before doing anything.

I will cover basically what I have done to 2 of mine.

This is NOT a complicated process, but a good dose of common sense goes a long way here.

Different markets will have different items, so if something is not on your car, move on, OR ask, just in case it is disguised as something else.

I went the whole hog, so if something mentioned is not what you want to do, again, move on, or ask. I basically used the Carby V12 as the work board, and that engine had zero emissions components outside the USA.

There are lots of small vac hoses all over the place, mainly in and around the RH throttle body, so systematically remove these, and plug the actual ports (spigots) to prevent vac leaks. The thermal/vac switches etc can stay in place so coolant stays inside the system.

The hoses that are in and around the LH throttle housing are mainly to do with the fuel tank venting system, and I suggest leaving them as is, but NEW hoses replacing the originals, doing ONE hose at a time, would be a smart idea. My fuel tanks are vented to atmosphere, so the charcoal canister and all those now useless valves etc are gone.

There is a crankcase breather system that causes a lot of issues. I remove that Y pipe, the one across the top of the engine, plug the 2 manifold spigots, and plug the grommet in the LH air cleaner backing plate (I used Black RTV). Remove the rubber elbow/snout thingy at the front of the LH cylinder head, under it is a cylinder style filter, remove that filter. If that rubber elbow is suspect (most are), replace it. Reconnect the pipe that goes from that elbow to the air cleaner housing.

Now YOUR head has to be really switched on:

The distributor will have a vac capsule on the side, with a vac spigot facing the rear of the engine. Attach a vac hose to this spigot, thread the rest of the hose through the inlet manifold and plug it onto one of the spigots UNDER the RH throttle body.

At the rear of the RH inlet manifold is a plate that has 4? spigots on it. One of these is for the transmission shift control modulator. That hose travels down behind the insulation board, so DON’T simply pull it out. Attach the new hose to the old hose and CAREFULLY drag the new hose thru whilst the old hose is being removed. This is a strong recommendation replacement, coz this hose can leak and cause trans issues and idle issues.

Another hose is for the interior vac controls of the a/c and heater system. It enters the cabin on the LH side from under the washer bottle on our RHD cars. Check this hose carefully, but it is usually OK.

The AAV, yes that bloody thing. It MUST close 100%, no near enough, no guess work. If it does not close 100% then engine idle will be a shocker, up and down, and will drive you to drink. Also, the 90deg elbow on top of that AAV is worse than the AAV. It is a marvel of British engineering, and they split with ease. Removing and replacing this elbow will test your sense of humour at best. I ALWAYS have a new elbow on hand BEFORE even touching the AAV. Leaks here will reek havoc.

There is also a 10mm vac hose coming off the centre of the 1” balance pipe. This is the vac signal hose connection for the ECU. Generally OK, but check it all the same.

The 2 hoses from the distributor cap/body side are important. NOT vac hoses at all, but MUST be connected, or the cap may explode from fume build up. One hose has a small plastic filter in its end, and hangs near the washer bottle. The 2nd hose goes to a plastic elbow inserted in the AAV intake elbow hose, thus “draft venting” the cap. Check it is installed, and that plastic elbow is not broken, most are, so replace it with another, which you will find at a hardware/irrigation supply outfit.

At the front of each inlet manifold are the FPR’s (Fuel Pressure Regulators), and they have a small vac hose attached. Make sure these are in good condition. I removed the RH regulator on mine, as it has NO fuel pressure use at all. It is supposed to be a damper of sorts for the pulse reaction of the injectors on the rest of the system, AND, to hold pressure in the rail for a small time after shutdown. In fact it causes more issues than it solves. Unstable idle, like an intermittent stumble, of the HE is mostly sorted with this unit removed.

Also at the front of each inlet manifold are the “over run valves”, concealed under alloy cup type castings. They are secured with 3 bolts each, 2 (one each side) of which are hollow, and are the vac supply for the FPR’s. These valves leak, badly, and idle is compromised. Jaguar deleted these, then returned them, the deleted them again. I remove them, and plate the ends of each manifold.

Some markets had a 15 minute timer system. This had a largish electric solenoid inserted in a grommet in the RH air cleaner backing plate  (towards the front), and opened a vac passage for “Extra Air” at start up, dependant on coolant temps. This can be removed, the wires taped and insulated, and the hose spigot capped.

The ECU is next, these HE cars came originally with a 6CU unit. This was updated to a 16CU unit somewhere in 1987, again, dependant on markets. The 6CU was known for issues with the fuel pump control circuit, and generally unreliable. The 16CU processes the data 5 times faster than the 6CU, is way more reliable, and a better all round unit. It is a plug and play replacement. Replacing it is NOT mandatory, but if a 16CU unit is found, I strongly suggest grabbing it and fitting it to your car.

Fuel enrichment now needs to be reinstalled, as it was probably removed with all the junk from the RH throttle body plumbing. The Blue/White valve with a vac spigot and 2 wires is the item I am talking about. Find a T piece, and insert it in that hose from the distributor, and attach a new piece of hose from it to the Blue/White valve spigot. Plug the 2 wires into the connectors that should be hanging somewhere near the micro switch on the throttle capstan, or down near 5A spark plug. If you cannot find the 2 wires, splice those valve wires into the 2 wires of that micro switch. Jaguar used either/or to enrich the fuel on acceleration.

Then we have the “Dump Valve”. Some markets did not get this thing. It is used to “Dump” vac from the vac advance hose instantly, rather than wait for it to “bleed off” when the accelerator is stomped on. This “bleed off” may take 1-2 seconds, and the engine COULD pre-ignite until that vac capsule is returned to zero position with the loss of vacuam. A second T in that same hose, BUT, on the engine side of the other T, will suffice. A hose goes FROM the 2nd T to the dump valve, and the 2nd spigot has a hose from it TO the rear plate of the inlet manifold, where there is s spare spigot. Check this valve if you have it, as they do fail and will leak vac, and that will affect idle also. The hose connections are:

Vac advance T is the lower port

Inlet manifold spigot in the top port, as in the port AT the flat face end of the dump valve.

Cleaning the inside bore of the throttle bodies in mandatory. Once the “Black Goo” builds up in here, the idle will become erratic.

I also open the throttle disc clearance to 0.003”, from the factory set 0.002”. It just seems to work better.

Refit the air filters and their covers. NEVER, EVER attempt to start one of these engines with the air filter, and or, covers OFF. These engines can backfire, and belch flames out of the throttle body, and apart from scaring the crap out of you, it can/will set fire to stuff in the area.

Start the engine, let it settle. If it is now running smooth and sweet, you have done well.

There will be some “trimming” of the distributor timing, that can be done by “drive timing” the engine. This is in a separate write up if required.

Also the idle fuel potentiometer, inside the ECU can be trimmed, which is also in a separate write up.