HE V12 Ignition amp resistors explained.

About the resistors, the white piece of rubber in the amp (also known as “The Blob”), has 2 resistors inside it.  One is 10k, the other 6.8k. They are connected at one end to terminal C of the module.  This same terminal goes to the coil -ve and the large diode (Zenor).

The other end of each resistor goes separately to tacho, and fuel ECU pin #18 (white/gray wires).  The 10K resistor connects to the tacho and the 6.8K connects to the EFI ECU.

On the ignition amp, both wires are the same colour but the tacho wire has a male terminal and the ECU wire has a female terminal. The wires they connect to are clearly distinguishable, since the ECU lead is a shielded cable and the tacho wire is a plain W/S/U wire.

You can easily test the resistors without even unbolting the amp from the manifold. Just disconnect the W/B wire from the ignition coil -ve and the leads to the ECU and tacho and measure from the W/B wire to each.  If the resistor feeding the tacho fails the only symptom would be a non-functioning tacho, but a failure of the resistor feeding the ECU will stop the engine with conviction.