Jaguar V12 A/C Idler and/or Fan Hub bearing replacement

This a rewrite, cyber space stole the original.

The bearing if the A?C idler arm, AND the Fan Hub are know as:

10 Integral Shaft Bearing

2) Water Pump Bearing.

Depending on where you live and the language spoken, I talk Aussie, so the #2 is Aussie speak.

The Jag bearing way back, 25+ years ago, was insane $, so I went looking.

My years in the Aftermarket Parts World helps a bucket load.

Water Pump Bearing is what it is, nothing exciting, and as I have stated thousands of times, “not much on Jaguar is Unique, they used what was readily available most times”.

The shaft lengths may vary, it is a POPULAR bearing, and used in many, many applications.

The one I used was simply called a “5/8 Water Pump Bearing”.

There is an image of the style at the bottom.

I had a lathe at the time, and simply sliced the shafts to match the old unit. If no lathe, and you dont know someone with one, a thin cutting disc in an angle grinder may suffice. That I will leave WELL ALONE, as I do not know.

The old one has a flat ground into the outer casing to give the grub screw something to “grip”. I simply used the Dremel, and ground a mirror of the old onto the new, 20 seconds.

The pulley hub on the old was simply pressed off, and reused. I applied 2 drops of Loctite Bearing Mount to just get that warm and fuzzy feeling.

Pressed the unit into the alloy arm, again, 2 drops of Loctite. The grub screw was refitted.

NOW PAY BLOODY ATTENTION.

That grub screw is tightened JUST OVER FINGER TIGHT. Any more, and that sucker will distort the outer casing, and screw the bearing, and you will start the process all over again until you PAY ATTENTION.

I am OLD (AKA “The Fossil”), and that “pay attention” is NEW language to me, as I grew up with COMMON SENSE, which is a talent and a phrase of a bygone era.

Fan Hub. The thing that the engine driven fan attaches to.

I dont run engine fans, they were removed very early in ownership, so I have not had the pleasure of replacing that bearing, but, it is the same bearing, as best I can establish.

You are smart, so can work that out using the above as a guide.

Figure 1: Water Pump Bearing