TPS wiring Bourns TO XJ40 3 wire TPS.

Before we start, my car has been completely REWIRED under the bonnet, so factory colours mean SQUAT, as does much of the factory electrical items that have either been removed, or relocated OUTSIDE the engine bay.

I did not note the TPS chassis/TPS wire colour changes, so this is a guide, and since you are smart/clever, you will sort it quickly.

The XJ40 TPS has 3 wires, Pink, Green, Black, simple.

On that TPS the Pink is at one end of the unit, the Green is the centre wire, and the Black is the other end.

The factory chassis plug has 3 wires, surprise surprise, and 2 of them have 2 wires each, coz they pick up the coolant temp sensor, and air temp sensor going back to the ECU. That probably surprises you also.

ALL these readings are with the ignition ON, and engine OFF.

The readings were done with the meter RED lead as the probe, and the meter BLACK lead attached to earth.

The TPS PINK is the 5v (or very close) feed FROM the ECU, and NO change from idle to WOT.

The TPS GREEN is 0.29v at idle and 4.9v at WOT (full throttle).

The TPS BLACK is 0.05v at both idle and WOT.

The probing for the 0.34v +/- is via the TPS Black and Green wires.