ADAPTIVE FRONT LIGHTING SYSTEM (AFS)

The AFS headlamp is similar in its construction to the Xenon, non-AFS headlamp. The projector

module is constructed and functions as described for the Xenon headlamp but with the addition of

the AFS system which allows the projector module to be moved vertically and horizontally. The AFS

system is controlled by a AFS control module which is integral with the headlamp levelling module.

The AFS headlamp assembly contains an additional carrier frame which provides location for the AFS

actuators. The remaining lamps are as described previously. The carrier frame is attached to the AFS

vertical actuator. The projector module has a central pivot point which allows the module to move

horizontally in response to operation of the AFS horizontal actuator.

The AFS actuators are bi-polar (2 phase) dc stepper motors which are driven by a power output from

the AFS power module located on the rear of the headlamp assembly. Each stepper motor receives

its position information from the AFS control module via the applicable AFS power module. When

the actuators are powered to their requested positions, a holding current is applied to maintain the

actuator position.

The actuators do not supply a positional feedback signal to the AFS control module. Each stepper

motor requires referencing each time the AFS system becomes active. When the AFS system is

active, each vertical actuator is driven to the low beam position and each horizontal actuator is

driven to an inboard position until a mechanical stop in the actuator is reached. Once the stop is

reached a step counter in the AFS control module is set to zero and the actuator is then powered to

the operating position as determined by the AFS control module software.

The AFS control module receives front and rear vehicle height data from the height sensors to

provide headlamp levelling adjustment via the vertical actuator motor. The AFS control module also

receives vehicle speed signals from the ABS module to adjust the projector module vertically to

increase the beam range as the vehicle speed increases.

| Item | Part Number | Description |
| --- | --- | --- |
| A |  | Conventional headlamp beam distribution |
| B |  | AFS swivel headlamp beam distribution |

AFS Control Module

The AFS control module is located on the bulkhead, at the bottom of the RH (right-hand) 'A' pillar.

The AFS control module is a dual functionality unit which also incorporates software to control the

headlamp leveling. The AFS control module is connected to the high speed CAN (controller area

network) bus and receives inputs from other vehicle systems on the status of the following

parameters:

· Steering angle

· Vehicle speed

· Headlamp status

· Engine running

· Reverse gear selected

· AUTO lights on.

The AFS will only operate when the AFS control module receives an engine running signal on the high speed CAN (controller area network) bus from the ECM (engine control module) . When the engine running signal is received the AFS control module performs its initialisation routine.

The AFS will also function when the lighting control switch is in the AUTO position and the AFS

control module receives a lights on signal from the light sensor and an engine running signal.

The AFS control module then monitors the inputs from the other vehicle systems to control the AFS

functionality according to cornering angles and vehicle speed.

The AFS control module is connected to each AFS power module on a private Local Interconnect

Network (LIN) bus. The power modules read operating values supplied from the AFS control module

and control the output drivers for the stepper motor actuators inside the headlamp assembly.

AFS Operation

The AFS controls the swiveling angle of each projector module using speed and steering angle signals. The angles of each projector module differ to give the correct spread of light, for example, when turning left, the LH (left-hand) projector module will have a greater swiveling angle than the RH (right-hand) projector module.

Initialisation Procedure

When the AFS control module receives an ignition mode on signal, the control module performs the

initialisation procedure which ensures that the headlamps are correctly aligned on both their vertical

and horizontal axes.

The headlamp leveling motors are powered from their current position, which can be either the

upper or lower limit or somewhere in between, to their lower position and then back to the 0

degrees position.

The AFS swivel initialisation starts less than 1 second after the headlamp leveling initialisation is

activated to ensure that the headlamps are at or below the 0 degree position in the vertical axis, thus preventing glare to oncoming vehicles. The AFS swivel initialisation is completed in less than 2.5

seconds. The LH (left-hand) and RH (right-hand) AFS actuator motors are powered from the 0 degree

position to their fully inboard position, then to their fully outboard position and then back to the 0

degree position.

Failure Mode

In the event of a failure of the AFS system, a warning indicator in the instrument cluster is illuminated to warn the driver. The AFS warning indicator illuminates when the ignition is in the ignition on mode and will flash continuously until the fault is rectified. The AFS warning indicator will also be illuminated if a failure of the steering angle sensor or the vehicle speed signal is detected.

Illumination of the warning indicator does not necessarily mean that there is a fault with the AFS

system. The fault may be caused by a failure of another system preventing the AFS system from

operating correctly.

The AFS control module performs a diagnostic routine every time AFS is requested. If any fault is

found, the AFS control module will suspend the operation of the AFS function.

If the AFS leveling system has failed with the projector module in a position other than the correct

straight ahead position, the AFS control module will attempt to drive the projector module to a

position a small amount lower than the standard position. If the swivel function has failed, the AFS

control module will lower the projector module using the leveling actuator to a position much lower

than standard to prevent excess glare to oncoming vehicles.

The AFS control module software can detect an internal failure of the control module control circuits.

The control module will power the projector modules to the zero position and prevent further

operation.

Faults can be investigated by interrogating the AFS control module using an approved Jaguar

diagnostic system to check for fault codes.

AUTOMATIC HEADLAMP OPERATION

The automatic headlamp function is a driver assistance system. The driver can override the system

operation by selection of side lamp or headlamp on if the ambient light conditions require front and

rear lighting to be active. The automatic headlamp system uses a light sensor and the AJB, which is

connected to the instrument cluster, via the medium speed CAN (controller area network) bus, to

control 'AUTO' headlamp functionality.

The light sensor is located in the centre of the instrument panel upper surface. The sensor has three

functions; houses the alarm system LED (light emitting diode) , auto lamp function for AUTO

headlamps and sun load sensor for the A/C (air conditioning) system.

The sensor is hardwired to the instrument cluster which supplies a 5V current via a pull-up resistor to 2 photodiode light sensors within the sensor housing. The ambient light signals are returned to the instrument cluster by the light sensor varying the current drawn from the 5V output, hence varying the voltage at the 'pull-up' resistor. The instrument cluster then outputs a message relating to the ambient light level on the medium speed CAN (controller area network) bus. The AJB receives the message and determines if AUTO lamps are required to be activated.

Operation of the 'AUTO' headlamps requires the ignition to be in the ignition mode and the lighting

control switch must be in the AUTO position. The AJB will only activate the AUTO headlamps on

receipt of a valid ambient light signal on the medium speed CAN (controller area network) bus from

the instrument cluster.

HEADLAMP LEVELING

Headlamp leveling provides for the automatic adjustment of the vertical aim of the headlamps. The

leveling system is required to minimise glare to other road users when the vehicle attitude changes

due to braking or acceleration.

Headlamp leveling is controlled by a headlamp leveling module which is located on the bulkhead, at

the bottom of the RH (right-hand) 'A' pillar.

NOTE:

On vehicles with the AFS system, the headlamp leveling software is incorporated into the AFS control module and the module is known as the AFS control module.

The headlamp leveling system comprises the following components and information from other

vehicle systems:

· Front and rear vehicle height sensors

· Two headlamp leveling, vertical adjustment motors

· Headlamp leveling module (or AFS control module if vehicle is fitted with AFS)

· Ignition in on mode

· Vehicle speed information form ABS (anti-lock brake system) module.

When the ignition is in the ignition power mode, power is supplied via the ignition relay in the

battery junction box to the headlamp leveling module (or AFS control module if vehicle is fitted with

AFS).

NOTE:

When the headlamp leveling module receives a power supply from the ignition relay, the

module performs an initialization procedure to correctly align the vertical aim of the headlamps and to determine if the leveling motors are functioning correctly.

The headlamp leveling module receives information relating to vehicle attitude from the front and

rear height sensors. The headlamp leveling module processes these signal and provides an output to

the headlamp leveling motors to adjust the headlamp vertical aim according to vehicle speed and

attitude.